Computer Networks and Internets

By: Douglas E. Comer

http://www.eg.bucknell.edu/~cs363/lecture_notes/lecture_notes.html

| CHAPTER | TITLE | PAGE |
| --- | --- | --- |
| Chapter 1 | Introduction | 2 |
| Chapter 2 | Motivation and Tools | 5 |
| PART I | Data Transmission |  |
| Chapter 3 | Transmission Media | 10 |
| Chapter 4 | Local Asynchronous Communication (RS-232) | 16 |
| Chapter 5 | Long-Distance Communication (Carriers And Modems) | 24 |
| PART II | Packet Transmission |  |
| Chapter 6 | Packets, Frames, And Error Detection | 35 |
| Chapter 7 | LAN Technologies And Network Topology | 45 |
| Chapter 8 | Hardware Addressing And Frame Type Identification | 61 |
| Chapter 9 | LAN Wiring, Physical Topology, And Interface Hardware | 70 |
| Chapter 10 | Extending LANs: Fiber Modems, Repeaters, Bridges, and Switches | 83 |
| Chapter 11 | Long-Distance Digital Connection Technologies | 94 |
| Chapter 12 | WAN Technologies And Routing | 103 |
| Chapter 13 | Network Ownership, Service Paradigm, And Performance |  |
| Chapter 14 | Protocols And Layering | 115 |
| PART III | Internetworking |  |
| Chapter 15 | Internetworking: Concepts, Architecture, and Protocols | 128 |
| Chapter 16 | IP: Internet Protocol Addresses | 134 |
| Chapter 17 | Binding Protocol Addresses (ARP) | 141 |
| Chapter 18 | IP Datagrams And Datagram Forwarding | 150 |
| Chapter 19 | IP Encapsulation, Fragmentation, And Reassembly | 156 |
| Chapter 20 | The Future IP (IPv6) | 162 |
| Chapter 21 | An Error Reporting Mechanism (ICMP) | 167 |
| Chapter 22 | TCP: Reliable Transport Service | 171 |
| PART IV | Network Applications |  |
| Chapter 23 | Client-Server Interaction | 183 |
| Chapter 24 | The Socket Interface | 189 |
| Chapter 25 | Example Of A Client And A Server | 196 |
| Chapter 26 | Naming With The Domain Name System | 201 |
| Chapter 27 | Electronic Mail Representation And Transfer | 211 |
| Chapter 28 | File Transfer And Remote File Access | 221 |
| Chapter 29 | World Wide Web Pages And Browsing | 227 |
| Chapter 30 | CGI Technology For Dynamic Web Documents | 233 |
| Chapter 31 | Java Technology For Active Web Documents | 239 |
| Chapter 32 | RPC and Middleware | 247 |
| Chapter 33 | Network Management (SNMP) | 252 |
| Chapter 34 | Network Security | 258 |
| Chapter 35 | Initialization (Configuration) | 262 |
| Bibliography |  |  |
|  | Prof. M. Anvari, OS and Networking |  |

Chapter 1 - Introduction

| Section | Title |
| --- | --- |
| 1 | How do Computer Networks and Internets Operate? |
| 2 | Explosive growth |
| 3 | Internet |
| 4 | Economic impact |
| 5 | Complexity |
| 6 | Abstractions and concepts |
| 7 | On-line resources |

How do Computer Networks and Internets Operate?

Network: system for connecting computer using a single transmission technology
Internet: set of networks connected by routers that are configured to pass traffic among any computers attached to networks in the set

- Data transmission - media, data encoding

- Packet transmission - data exchange over a network

- Internetworking - universal service over a collection of networks

- Network applications - programs that use an internet

Explosive growth

- New phenomenon - now, networks are an important part of everyday activities

  - Business

  - Home

  - Government

  - Education

- Global Internet growing exponentially

  - Initially a research project with a few dozen sites

  - Today, millions of computers and thousands of networks world-wide

Internet

- Roots in military network called Arpanet

  - Fundamental changes from centralized to distributed computing

  - Incorporated features for reliability and robustness

    - Multiple links

    - Distributed routing

- Ethernet made local networking feasible

- TCP/IP protocol made internetworking possible

  - Developed after Arpanet

  - Switchover occurred in 1983

- Exponential growth - doubling every 18 months

Economic impact

- Large industry has grown around:

  - Networking hardware

  - Computers

  - Software

- Companies must integrate planning, implementation, management and upgrade

Complexity

- Computer networking is complex

  - Many different hardware technologies

  - Many different software technologies

  - All can be interconnected in an internet

- No underlying theory

- Terminology can be confusing

  - TLAs

  - Industry redefines or changes terminology from academia

  - New terms invented all the time

Abstractions and concepts

- Will concentrate on abstractions and concepts to unravel complexity

- Examples:

  - Types of LAN wiring, rather than details of LAN data transmission

  - Definition and concept of congestion, rather than specific congestion control mechanisms

Chapter 2 - Motivation and Tools

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Historic motivation |
| 3 | ARPA |
| 4 | Packet switching |
| 5 | Internetworking |
| 6 | History and growth |
| 7 | Growth since 1981 |
| 8 | Growth (logarithmic axis) |
| 9 | Probing the Internet |
| 10 | ping |
| 11 | ping (Example) |
| 12 | ping (Example) |
| 13 | traceroute |
| 14 | traceroute (Example) |
| 15 | traceroute (Example) |
| 16 | Web access to tools |

Introduction

- Motivation

- Service

- Tools for exploration

Historic motivation

- Early computers were expensive

  - Large footprint

  - Centralized

- Programs took a long time to run

- Couldn't afford to put computers everywhere

ARPA

- Advanced Research Projects Agency initiated project to connect researchers with computers

- Adopted new technology:

  - Packet switching

  - Internetworking

- Resulted in system for remote access to expensive resources

Packet switching

- Data transmitted in small, independent pieces

  - Source divides outgoing messages into packets

  - Destination recovers original data

- Each packet travels independently

  - Includes enough information for delivery

  - May follow different paths

  - Can be retransmitted if lost

Internetworking

- Many (mutually incompatible) network technologies

- No one technology appropriate for every situation

- Internetworking glues together networks of dissimilar technologies with routers

- Result is virtual network whose details are invisible

History and growth

- ARPAnet began in late 1960s (not using TCP/IP)

- TCP/IP developed in late 1970s

- ARPAnet switched to TCP/IP in early 80s

- Start of Internet

  - Few hundred computers

  - Few tens of networks

Growth since 1981

Probing the Internet

- Two tools:

  - ping - sends message that is echoed by remote computer

  - traceroute - reports path to remote computer

ping

- Sends packet to remote computer

- Remote computer replies with echo packet

- Local computer reports receipt of reply

- % ping merlin.cs.purdue.edu

- merlin.cs.purdue.edu is alive

ping (Example)

- Can arrange to send multiple packets

- Reports round trip time

- % ping -s www.bucknell.edu 100 5

- PING web.bucknell.edu: 100 data bytes

- 108 bytes from web.bucknell.edu (134.82.6.6): icmp_seq=0. time=8. ms

- 108 bytes from web.bucknell.edu (134.82.6.6): icmp_seq=1. time=5. ms

- 108 bytes from web.bucknell.edu (134.82.6.6): icmp_seq=2. time=3. ms

- 108 bytes from web.bucknell.edu (134.82.6.6): icmp_seq=3. time=2. ms

- 108 bytes from web.bucknell.edu (134.82.6.6): icmp_seq=4. time=2. ms

- ----web.bucknell.edu PING Statistics----

- 5 packets transmitted, 5 packets received, 0% packet loss

- round-trip (ms)  min/avg/max = 2/4/8

ping (Example)

% PING merlin.cs.purdue.edu: 100 data bytes

108 bytes from merlin.cs.purdue.edu (128.10.2.3): icmp_seq=0. time=64. ms

108 bytes from merlin.cs.purdue.edu (128.10.2.3): icmp_seq=1. time=58. ms

108 bytes from merlin.cs.purdue.edu (128.10.2.3): icmp_seq=2. time=55. ms

108 bytes from merlin.cs.purdue.edu (128.10.2.3): icmp_seq=3. time=63. ms

108 bytes from merlin.cs.purdue.edu (128.10.2.3): icmp_seq=4. time=54. ms

----merlin.cs.purdue.edu PING Statistics----

5 packets transmitted, 5 packets received, 0% packet loss

round-trip (ms)  min/avg/max = 54/58/64

traceroute

- Sends series of packets along path to destination

  - Each successive packet identifies next router along path

  - Uses expanding ring search

- Reports list of packets

Traceroute (Example)

% traceroute www.bucknell.edu

traceroute to web.bucknell.edu (134.82.6.6), 30 hops max, 40 byte packets

1  DanaRout-p13-s56.eg.bucknell.edu (134.82.56.254)  8 ms  5 ms  5 ms

2  CCSServB-p1p17-s254.bucknell.edu (134.82.254.3)  4 ms  7 ms  4 ms

3  web.bucknell.edu (134.82.6.6)  3 ms  3 ms  3 ms

traceroute (Example)

traceroute merlin.cs.purdue.edu

traceroute to merlin.cs.purdue.edu (128.10.2.3), 30 hops max, 40 byte packets

1  CCSServC (134.82.7.254)  2 ms  1 ms  1 ms

2  134.82.254.253 (134.82.254.253)  2 ms  2 ms  3 ms

3  12.127.210.89 (12.127.210.89)  22 ms  20 ms  20 ms

4  gr1-a3100s5.wswdc.ip.att.net (192.205.34.9)  20 ms  20 ms  20 ms

5  Hssi2-1-0.GW1.DCA1.ALTER.NET (157.130.32.21)  20 ms  20 ms  20 ms

6  104.ATM2-0.XR2.DCA1.ALTER.NET (146.188.161.30)  21 ms  39 ms  20 ms

7  194.ATM2-0.TR2.DCA1.ALTER.NET (146.188.161.146)  20 ms  20 ms  20 ms

8  101.ATM6-0.TR2.CHI4.ALTER.NET (146.188.136.109)  40 ms  41 ms  56 ms

9  198.ATM7-0.XR2.CHI4.ALTER.NET (146.188.208.229)  41 ms  41 ms  41 ms

10  194.ATM8-0-0.GW1.IND1.ALTER.NET (146.188.208.165)  63 ms  66 ms  51 ms

11  purdue-gw.customer.alter.net (157.130.101.106)  56 ms  54 ms  54 ms

12  cisco-cs-atm.gw.purdue.edu (128.210.252.21)  66 ms  65 ms  63 ms

13  merlin.cs.purdue.edu (128.10.2.3)  68 ms  84 ms  63 ms

Web access to tools

ping/traceroute

http://www.net.cmu.edu/cgi-bin/netops.cgi

Ping/Traceroute Gateway
Ping/Traceroute Gateway
This application allows you to ping or traceroute to hosts on other networks. Carnegie Mellon has redundant internet connections described here. We also have a connection to the VBNS.

Top of Form

Host (IP or hostname): 
Operation: traceroute ping

Chapter 3 - Transmission Media

| Section | Title |
| --- | --- |
| 1 | Basic Idea |
| 2 | Transmission media |
| 3 | Copper wires |
| 4 | Glass fibers |
| 5 | Radio |
| 6 | Wireless Example |
| 7 | Wireless Exmaple |
| 8 | Microwave |
| 9 | Infrared |
| 10 | Laser |
| 11 | Choosing a medium |
| 12 | Media in use at Bucknell |

Basic Idea

- Encode data as energy and transmit energy

- Decode energy at destination back into data

- Energy can be electrical, light, radio, sound, ...

- Each form of energy has different properties and requirements for transmission

Transmission media

- Transmitted energy is carried through some sort of medium

- Transmitter encodes data as energy and transmits energy through medium

  - Requires special hardware for data encoding

  - Requires hardware connection to transmission medium

- Media can be copper, glass, air, ...

Copper wires

- Twisted pair uses two wires

- Coaxial cable includes shield for improved performance

Glass fibers

- Thin glass fiber carries light with encoded data

- Plastic jacket allows fiber to bend (some!) without breaking

- Fiber is very clear and designed to reflect light internally for efficient transmission

- Light emitting diode (LED) or laser injects light into fiber

- Light sensitive receiver at other end translates light back into data

Radio

- Data transmitted using radio waves

- Energy travels through the air rather than copper or glass

- Conceptually similar to radio, TV, cellular phones

- Can travel through walls and through an entire building

- Can be long distance or short distance

  - Long distance with satellite relay

  - Short distance - wireless computer network

Wireless Example

- Wireless bridge and antenna

Wireless Example

- Remote station (laptop) interface

Microwave

- High frequency radio waves

- Unidirectional, for point-to-point communication

- Antennas mounted on towers relay transmitted data

Infrared

- Infrared light transmits data through the air

- Similar to technology used in TV remote control

- Can propagate throughout a room (bouncing off surfaces), but will not penetrate walls

- Becoming common in personal digital assistants

Laser

- Unidirectional, like microwave

- Higher speed than microwave

- Uses laser transmitter and photo-sensitive receiver at each end

- Point-to-point, typically between buildings

- Can be adversely affected by weather

Choosing a medium

- Copper wire is mature technology, rugged and inexpensive; maximum transmission speed is limited

- Glass fiber:

  - Higher speed

  - More resistant to electro-magnetic interference

  - Spans longer distances

  - Requires only single fiber

  - More expensive; less rugged

- Radio and microwave don't require physical connection

- Radio and infrared can be used for mobile connections

- Laser also does not need physical connection and supports higher speeds

Media in use at Bucknell

- Copper/fiber for long-distance connection to Internet

- Fiber between buildings

- Copper within buildings

Chapter 4 - Local Asynchronous Communication

Last modified: Mon Jan 18 08:51:15 2000

| Section | Title |
| --- | --- |
| 1 | Bit-wise data transmission |
| 2 | Asynchronous communication |
| 3 | Using electric current to send bits |
| 4 | Sending bits - example |
| 5 | Transmission timing |
| 6 | RS-232 |
| 7 | Details of RS-232 |
| 8 | RS-232 wiring and connectors |
| 9 | Identifying asynchronous characters |
| 10 | Timing |
| 11 | Measures of transmission rates |
| 12 | Framing |
| 13 | Full-duplex communication |
| 14 | RS-232 connection standards |
| 15 | 2-3 swap |
| 16 | RS-232 cable breakout-box |
| 17 | Limitations of real hardware |
| 18 | Hardware bandwidth |
| 19 | Bandwidth and data transmission |
| 20 | Summary |

Bit-wise data transmission

- Data transmission requires:

  - Encoding bits as energy

  - Transmitting energy through medium

  - Decoding energy back into bits

- Energy can be electric current, radio, infrared, light

- Transmitter and receiver must agree on encoding scheme and transmission timing

Asynchronous communication

- One definition of asynchronous: transmitter and receiver do not explicitly coordinate each data transmission

  - Transmitter can wait arbitrarily long between transmissions

  - Used, for example, when transmitter such as a keyboard may not always have data ready to send

- Asynchronous may also mean no explicit information about where data bits begin and end

Using electric current to send bits

- Simple idea - use varying voltages to represent 1s and 0s

- One common encoding use negative voltage for 1 and positive voltage for 0

- In following figure, transmitter puts positive voltage on line for 0 and negative voltage on line for 1

Transmission timing

- Encoding scheme leaves several questions unanswered:

  - How long will voltage last for each bit?

  - How soon will next bit start?

  - How will the transmitter and receiver agree on timing?

- Standards specify operation of communication systems

  - Devices from different vendors that adhere to the standard can interoperate

  - Example organizations:

    - International Telecommunications Union (ITU)

    - Electronic Industries Association (EIA)

    - Institute for Electrical and Electronics Engineers (IEEE)

RS-232

- Standard for transfer of characters across copper wire

- Produced by EIA

- Full name is RS-232-C

- RS-232 defines serial, asynchronous communication

  - Serial - bits are encoded and transmitted one at a time (as opposed to parallel transmission)

  - Asynchronous - characters can be sent at any time and bits are not individually synchronized

Details of RS-232

- Components of standard:

  - Connection must be less than 50 feet

  - Data represented by voltages between +15v and -15v

  - 25-pin connector, with specific signals such as data, ground and control assigned to designated pins

  - Specifies transmission of characters between, e.g., a terminal and a modem

- Transmitter never leaves wire at 0v; when idle, transmitter puts negative voltage (a 1) on the wire

RS-232 wiring and connectors

Identifying asynchronous characters

- Transmitter indiciates start of next character by transmitting a zero

  - Receiver can detect transition as start of character

  - Extra zero called the start bit

- Transmitter must leave wire idle so receiver can detect transition marking beginning of next character

  - Transmitter sends a one after each character

  - Extra one call the stop bit

- Thus, character represented by 7 data bits requires transmission of 9 bits across the wire

- RS-232 terminology:

  - MARK is a negative voltage (== 1)

SPACE is a positive voltage (== 0)

Timing

- Transmitter and receiver must agree on timing of each bit

- Agreement accomplished by choosing transmission rate

  - Measured in bits per second

  - Detection of start bit indicates to receiver when subsequent bits will arrive

- Hardware can usually be configured to select matching bit rates

  - Switch settings

  - Software

  - Auto detection

Measures of transmission rates

- Baud rate measures number of signal changes per second

- Bits per second measures number of bits transmitted per second

- In RS-232, each signal change represents one bit, so baud rate and bits per second are equal

- If each signal change represents more than one bit, bits per second may be greater than baud rate

Framing

- Start and stop bits represent framing of each character

- If transmitter and reciver are using different speeds, stop bit will not be received at the expected time

- Problem is called a framing error

- RS-232 devices may send an intentional framing error called a BREAK

Full-duplex communication

- Two endpoints may send data simultaneously - full-duplex communication

- Requires an electrical path in each direction

RS-232 connection standards

- RS-232 specifies use of 25 pin connector (DB-25)

- Pins are assigned for use as data, ground and control:

  - Pin 2 - Receive (RxD)

  - Pin 3 - Transmit (TxD)

  - Pin 4 - Ready to send (RTS)

  - Pin 5 - Clear to send (CTS)

  - Pin 7 - Ground

2-3 swap

- Cable must cross-over wires to connect pins 2 and 3 on receiver and transmitter

- To avoid 2-3 swap, RS-232 specifies that modems transmit on pin 2 and receive on pin 3, while computers transmit on pin 3 and receive on pin 2

- However, RS-232 cables between two computers must have 2-3 swap

RS-232 cable breakout-box

- May need to test RS-232 connections

- Breakout-box gives access to signals

Limitations of real hardware

- Effects of wire mean waveforms look like:

- Longer wire, external interference may make signal look even worse

- RS-232 standard specifies how precise a waveform the transmitter must generate, and how tolerant the receiver must be of imprecise waveform

Hardware bandwidth

- Limitations on time to change voltages imposes upper limit on number of changes per second

- Theoretical upper limit is called the bandwidth

- Measured in cycles per second or Hertz

Bandwidth and data transmission

- Nyquist sampling theorem expresses relationship between bandwidth and maximum data transmission speed

- For RS-232, using two voltages, maximum speed over medium with bandwidth B is 2B

- In general, for system using K different states, maximum is 2Blog2K

- In practice, noise limits maximum data transmission rate to less than maximum allowed by Nyquist sampling theorem; Claude Shannon

Summary

- Asynchronous communication - data can start at any time; individual bits not delineated

- RS-232 - EIA standard for asynchronous character transmission

- Characters per second and baud rate

- Bandwidth limits maximum data transmission rate

Chapter 5 - Long-Distance Communication

Last modified: Wed Jan 20 07:28:26 2000

| Section | Title |
| --- | --- |
| 1 | Long-distance communication |
| 2 | Sending signals long distances |
| 3 | Oscillating signals |
| 4 | Encoding data with a carrier |
| 5 | Types of modulation |
| 6 | Examples of modulation techniques |
| 7 | Encoding data with phase shift modulation |
| 8 | Hardware for data transmission |
| 9 | Full duplex communication |
| 10 | Modems |
| 11 | Other types of modems |
| 12 | Leased serial data circuits |
| 13 | Optical, radio and dialup modems |
| 14 | Dialup modems |
| 15 | Operation of dialup modems |
| 16 | Carrier frequencies and multiplexing |
| 17 | Multiplexing |
| 18 | Spread spectrum multiplexing |
| 19 | Time division multiplexing |
| 20 | Summary |

Long-distance communication

- Encoding used by RS-232 cannot work in all situations

  - Over long distances

  - Using existing systems like telephone

- Different encoding strategies needed

Sending signals long distances

- Electric current becomes weaker as it travels on wire

- Resulting signal loss may prevent accurate decoding of data

- Signal loss prevents use of RS-232 over long distances

Oscillating signals

- Continuous, oscillating signal will propagate farther than electric current

- Long distance communication uses such a signal, called a carrier

- Waveform for carrier looks like:

- Carrier can be detected over much longer distances than RS-232 signal

Encoding data with a carrier

- Modifications to basic carrier encode data for transmission

- Technique called modulation

- Same idea as in radio, television transmission

- Carrier modulation used with all types of media - copper, fiber, radio, infrared, laser

Types of modulation

- Amplitude modulation - strength, or amplitude of carrier is modulated to encode data

- Frequency modulation - frequency of carrier is modulated to encode data

- Phase shift modulation - changes in timing, or phase shifts encode data

Examples of modulation techniques

- Amplitude modulation:

- Phase shift modulation:

Encoding data with phase shift modulation

- Amount of phase shift can be precisely measured

  - Measures how much of sine wave is "skipped"

  - Example shows 1/2 and 3/4 cycle

- Each phase shift can be used to carry more than one bit; in example, four possible phase shifts encode 2 bits:

  - 00 - no shift

  - 01 - 1/4 phase

  - 10 - 1/2 phase

  - 11 - 3/4 phase

- Thus, each phase shift carries 2 bits

- Data rate is twice the baud rate

Hardware for data transmission

- Modulator encodes data bits as modulated carrier

- Demodulator decodes bits from carrier

- Data transmission requires modulator at source and demodulator at destination

Full duplex communication

- Most systems provide for simultaneous bidirectional, or full duplex, transmission

- Requires modulator and demodulator at both endpoints:

- Long-distance connection is called 4-wire circuit

- Modulator and demodulator typically in single device called a modem (modulator/demodulator)

Modems

- If external to computer, RS-232 can be used between modem and computer

- If internal, direct bus connection used

- Can also be rack-mounted

Other types of modems

- ISDN modem

- Cable modem (front)

- Cable modem (rear) with coax connector for cable and 10Base-T connector

Leased serial data circuits

- Organizations often include 4-wire circuits in network

- Within a site - on a campus - organization can install its own 4-wire circuits

- Telephone company supplies off-campus wires

  - Telephone cables have extra wires (circuits) for expansion

  - Telephone company lease right to use wires to organization

  - Organization uses modems for data transfer

- Called serial data circuit or serial line

- Operates in parallel with (but not connected to) telephone circuits

Optical, radio and dialup modems

- Modems used with other media in addition to dedicated data circuits

- Special form of encoding/decoding transducers that use modulation for data encoding

  - Glass - data encoded as modulated light beam

  - Radio - data encoded as modulated radio signal

  - Dialup - data encoded as modulated sound

- Dialup modem connects to ordinary phone line

Dialup modems

- Circuitry for sending data

- Circuitry to mimic telephone operation

  - Lifting handset

  - Dialing

  - Replacing handset (hanging up)

  - Detect dial tone

- Full duplex on one voice channel

  - Different carrier frequencies for each direction

  - Filters eliminate interference

Operation of dialup modems

- Receiving modem waits for call in answer mode

- Other modem, in call mode:

  - Simulates lifting handset

  - Listens for dial tone

  - Sends tones (or pulses) to dial number

- Answering modem:

  - Detects ringing

  - Simulates lifting handset

  - Sends carrier

- Calling modem:

  - Sends carrier

- Data exchanged

Carrier frequencies and multiplexing

- Multiple signals with data can be carried on same medium without interference

  - Allows multiple simultaneous data streams

  - Dialup modems can carry full-duplex data on one voice channel

- Example - multiple TV stations in air medium

- Each separate signal is called a channel

Multiplexing

- Carrying multiple signals on one medium is called multiplexing

- Frequency division multiplexing (FDM) achieves multiplexing by using different carrier frequencies

- Receiver can "tune" to specific frequency and extract modulation for that one channel

  - Frequencies must be separated to avoid interference

  - Only useful in media that can carry multiple signals with different frequencies - high-bandwidth required

Spread spectrum multiplexing

- Spread spectrum uses multiple carriers

- Single data stream divided up and sent across different carriers

- Can be used to bypass interference or avoid wiretapping

Time division multiplexing

- Time division multiplexing uses a single carrier and sends data streams sequentially

- Transmitter/receiver pairs share single channel

- Basis for most computer networks used shared media - will give details in later chapters

Summary

- Long-distance communications use carrier and modulation for reliable communication

- Modulator encodes data and demodulator decodes data

- Can use amplitude, frequency or phase shift modulation

- Multiple transmitter/receiver pairs can use multiplexing to share a single medium

Chapter 6 - Packets, Frames and Error Detection

| Section | Title |
| --- | --- |
| 1 | Shared communication media |
| 2 | Packets |
| 3 | Motivation |
| 4 | Dedicated network access |
| 5 | Packet switching access |
| 6 | Time-division multiplexing |
| 7 | Time-division multiplexing - example |
| 8 | Packets and frames |
| 9 | Frame formats |
| 10 | Defining the framing standard |
| 11 | Frame format |
| 12 | Packet framing |
| 13 | Framing in practice |
| 14 | Transmitting arbitrary data |
| 15 | Data stuffing |
| 16 | Byte stuffing |
| 17 | Byte stuffing example |
| 18 | Transmission errors |
| 19 | Error detection and correction |
| 20 | Parity checking |
| 21 | Parity and error detection |
| 22 | Limitations to parity checking |
| 23 | Alternative error detection schemes |
| 24 | Checksums |
| 25 | Implementing checksum computation |
| 26 | Limitations to checksums |
| 27 | Cyclic redundancy checks |
| 28 | Hardware components |
| 29 | CRC hardware |
| 30 | Error detection and frames |
| 31 | Summary |

Shared communication media

- Most network use shared media which interconnect all computers

- However - only one source can transmit data at a time

Packets

- Most networks divide into small blocks called packets for transmission

- Each packet sent individually

- Such networks are called packet networks or packet switching networks

Motivation

- Coordination - helps transmitter and receiver determine which data have been received correctly and which have not

- Resource sharing - allows multiple computers to share network infrastructure

- Networks enforce fair use - each computer can only send one packet at a time

Dedicated network access

- 5MB file transferred across network with 56Kbps capacity will require 12 minutes:

| 5x106 bytes * 8 bits/byte 60 secs/minute * 56x103 bits/second | = 11.9 minutes |
| --- | --- |

- All other computers will be forced to wait 12 minutes before initiating other transfers

Packet switching access

- If file is broken into packets, other computers must only wait until packet (not entire file) has been sent

- From previous example, suppose file is broken into 1000 byte packets

- Each packet takes less than .2 seconds to transmit:

| 1000 bytes * 8 bits/byte 56x103 bits/second | = .143 seconds |
| --- | --- |

- Other computer must only wait .143 seconds before beginning to transmit

- Note:

  - If both files are both 5MB long, each now takes 24 minutes to transmit

  - BUT if second file is only 10KB long, it will be transmitted in only 2.8 seconds, while 5MB file still takes roughly 12 minutes

Time-division multiplexing

- Dividing data into small packets allows time-division multiplexing

- Each packet leaves the source and is switched onto the shared communication channel through a multiplexor

- At the destination, the packet is switched through a demultiplexor to the destination

Time-division multiplexing - example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap06/chap06_7.html

Shockwave

Packets and frames

- Packet is ``generic'' term that refers to a small block of data

- Each hardware technology uses different packet format

- Frame or hardware frame denotes a packet of a specific format on a specific hardware technology

Frame formats

- Need to define a standard format for data to indicate the beginning and end of the frame

- Header and trailer used to ``frame'' the data

Defining the framing standard

- Can choose two unused data values for framing

- E.g., if data is limited to printable ASCII, can use

  - ``Start of header'' (soh)

  - ``end of text'' (eot)

- Sending computer sends soh first, then data, finally eot

- Receiving computer interprets and discards soh, stores data in buffer and interprets and discards eot

Frame format

Packet framing

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap06/chap06_12.html

Shockwave

Framing in practice

- Incurs extra overhead - soh and eot take time to transmit, but carry no data

- Accommodates transmission problems:

  - Missing eot indicates sending computer crashed

  - Missing soh indicates receiving computer missed beginning of message

  - Bad frame is discarded

Transmitting arbitrary data

- Suppose system can't afford to reserve two special characters for framing

- E.g., transmitting arbitrary 8-bit binary data

- soh and eot as part of data will be misinterpreted as framing data

- Sender and receiver must agree to encode special characters for unambiguous transmission

Data stuffing

- Bit stuffing and byte stuffing are two techniques for inserting extra data to encode reserved bytes

- Byte stuffing translates each reserved byte into two unreserved bytes

- For example, can use esc as prefix, followed by x for soh, y for eot and z for esc:

Byte stuffing

- Sender translates each reserved byte into the appropriate encoding pair of bytes

- Receiver interprets pairs of bytes and stores encoded byte in buffer

- Data still framed by soh and eot

Byte stuffing example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap06/chap06_17.html

Shockwave

Transmission errors

- External electromagnetic signals can cause incorrect delivery of data

  - Data can be received incorrectly

  - Data can be lost

  - Unwanted data can be generated

- Any of these problems are called transmission errors

Error detection and correction

- Error detection - send additional information so incorrect data can be detected and rejected

- Error correction - send additional information so incorrect data can be corrected and accepted

Parity checking

- Parity refers to the number of bits set to 1 in the data item

  - Even parity - an even number of bits are 1

  - Odd parity - an odd number of bits are 1

- A parity bit is an extra bit transmitted with a data item, chose to give the resulting bits even or odd parity

  - Even parity - data: 10010001, parity bit 1

  - Odd parity - data: 10010111, parity bit 0

Parity and error detection

- If noise or other interference introduces an error, one of the bits in the data will be changed from a 1 to a 0 or from a 0 to a 1

- Parity of resulting bits will be wrong

  - Original data and parity: 10010001+1 (even parity)

  - Incorrect data: 10110001+1 (odd parity)

- Transmitter and receiver agree on which parity to use

- Receiver detects error in data with incorrect parity

Limitations to parity checking

- Parity can only detect errors that change an odd number of bits

  - Original data and parity: 10010001+1 (even parity)

  - Incorrect data: 10110011+1 (even parity!)

- Parity usually used to catch one-bit errors

Alternative error detection schemes

- Many alternative schemes exist

  - Detect multi-bit errors

  - Correct errors through redundant information

- Checksum and CRC are two widely used techniques

Checksums

- Sum of data in message treated as array of integers

- Can be 8-, 16- or 32-bit integers

- Typically use 1s-complement arithmetic

- Example - 16-bit checksum with 1s complement arithmetic

Implementing checksum computation

- Easy to do - uses only addition

- Fastest implementations of 16-bit checksum use 32-bit arithmetic and add carries in at end

- Can also speed computation by unrolling loop and similar optimizations

Limitations to checksums

- May not catch all errors

Cyclic redundancy checks

- Consider data in message as coefficients of a polynomial

- Divide that coefficient set by a known polynomial

- Transmit remainder as CRC

  - Good error detection properties

  - Easy to implement in hardware

Hardware components

CRC hardware

Error detection and frames

- Error detection typically done for each frame

- Error in frame typically causes receiver to discard frame

- Example - CRC sent after end of frame computed on data in frame

Summary

- Computer networks divide data into packets

  - Resource sharing

  - Fair allocation

- Hardware frames are specific to a particular hardware network technology

- Each frame has a specific format that identifies the beginning and end of the frame

- Error detection and correction is used to identify and isolate transmission errors

Chapter 7 - LAN Technologies and Network Topology

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Direct point-to-point communication |
| 3 | Connections in a point-to-point network |
| 4 | Connections in a point-to-point network |
| 5 | Reducing the number of communication channels |
| 6 | Growth of LAN technologies |
| 7 | Locality of reference |
| 8 | LAN topologies |
| 9 | Star topology |
| 10 | Star topology in practice |
| 11 | Ring topology |
| 12 | Bus topology |
| 13 | Why multiple topologies? |
| 14 | Ethernet |
| 15 | Ethernet speeds |
| 16 | Ethernet operation |
| 17 | Ethernet example |
| 18 | CSMA |
| 19 | CSMA example |
| 20 | Collision detection - CD |
| 21 | Collision example |
| 22 | Ethernet CD |
| 23 | Recovery from collision |
| 24 | Exponential backoff |
| 25 | Wireless LAN |
| 26 | Limited connectivity with wireless |
| 27 | CSMA/CA |
| 28 | Collisions |
| 29 | LocalTalk |
| 30 | Token ring |
| 31 | Transmission around a token ring |
| 32 | Using the token |
| 33 | Token and synchronization |
| 34 | IBM token ring |
| 35 | FDDI |
| 36 | FDDI and reliability |
| 37 | ATM - Star network |
| 38 | ATM details |
| 39 | ATM switches |
| 40 | Summary |

Introduction

- Sending packets across shared networks

- Network wiring topologies

- Details of Local Area Network (LAN) technologies

Direct point-to-point communication

- Computers connected by communication channels that each connect exactly two computers

- Forms mesh or point-to-point network

- Allows flexibility in communication hardware, packet formats, etc.

- Provides security and privacy because communication channel is not shared

Connections in a point-to-point network

- Number of wires grows as square of number of computers

- For N computers:

| Connections = | (n2 - n) 2 |
| --- | --- |

Connections in a point-to-point network

- Connections between buildings can be prohibitive:

- Adding a new computer requires N - 1 new connections

Reducing the number of communication channels

- LANs developed in late 1960s and early 1970s

- Key idea - reduce number of connections by sharing connections among many computers

  - Computers take turns - TDM

  - Must include techniques for synchronizing use

Growth of LAN technologies

- LAN technologies reduce cost by reducing number of connections

- But ... attached computers compete for use of shared connection

- Local communication almost exclusively LAN

- Long distance almost exclusively point-to-point

  - SMDS

  - ATM

Locality of reference

- Principle of locality of reference helps predict computer communication patterns:

  - Spatial (or physical) locality of reference - computers likely to communicate with other computers that are located nearby

  - Temporal locality of reference - computers are likely to communicate with the same computers repeatedly

- Thus - LANs are effective because of spatial locality of reference, and temporal locality of reference may give insight into which computers should be on a LAN

LAN topologies

- Networks may be classified by shape

- Three most popular:

  - Star

  - Ring

  - Bus

Star topology

- All computers attach to a central point:

- Center of star is sometimes called a hub

Star topology in practice

- Previous diagram is idealized; usually, connecting cables run in parallel to computers:

- Result is:

Ring topology

- Computers connected in a closed loop

- First passes data to second, second passes data to third, and so on

- In practice, there is a short connector cable from the computer to the ring

- Ring connections may run past offices with connector cable to socket in the office:

Bus topology

- Single cable connects all computers

- Each computer has connector to shared cable

- Computers must synchronize and allow only one computer to transmit at a time

Why multiple topologies?

- Each has advantages and disadvantages:

  - Ring ease synchronization; may be disabled if any cable is cut

  - Star easier to manage and more robust; requires more cables

  - Bus requires fewer cables; may be disable if cable is cut

- Bucknell has used all three; now almost entirely star topology

- Industry is settling on star topology as most widely used

Ethernet

- Widely used LAN technology

  - Invented at Xerox PARC (Palo Alto Research Center) in 1970s

  - Defined in a standard by Xerox, Intel and Digital - DIX standard

  - Standard now managed by IEEE - defines formats, voltages, cable lengths, ...

- Uses bus topology

  - Single coax cable - the ether

  - Multiple computers connect

- One Ethernet cable is sometimes called a segment

  - Limited to 500 meters in length

  - Minimum separation between connections is 3 meters

Ethernet speeds

- Originally 3Mbps

- Current standard is 10Mbps

- Fast Ethernet operates at 100Mbps

Ethernet operation

- One computer transmits at a time

- Signal is a modulated carrier which propagates from transmitter in both directions along length of segment

Ethernet example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap07/chap07_17.html

Shockwave

CSMA

- No central control managing when computers transmit on ether

- Ethernet employs CSMA to coordinate transmission among multiple attached computers

- Carrier Sense with Multiple Access

  - Multiple access - multiple computers are attached and any can be transmitter

  - Carrier sense - computer wanting to transmit tests ether for carrier before transmitting

CSMA example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap07/chap07_19.html

Shockwave

Collision detection - CD

- Even with CSMA, two computers may transmit simultaneously

  - Both check ether at same time, find it idle, and begin transmitting

  - Window for transmission depends on speed of propagation in ether

- Signals from two computers will interfere with each other

- Overlapping frames is called a collision

  - No harm to hardware

  - Data from both frames is garbled

Collision example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap07/chap07_21.html

# Shockwave

Ethernet CD

- Ethernet interfaces include hardware to detect transmission

  - Monitor outgoing signal

  - Garbled signal is interpreted as a collision

- After collision is detected, computer stops transmitting

- So, Ethernet uses CSMA/CD to coordinate transmissions

Recovery from collision

- Computer that detects a collision sends special signal to force all other interfaces to detect collision

- Computer then waits for ether to be idle before transmitting

  - If both computers wait same length of time, frames will collide again

  - Standard specifies maximum delay, and both computers choose random delay less than maximum

- After waiting, computers use carrier sense to avoid subsequent collision

  - Computer with shorter delay will go first

  - Other computers may transmit first

Exponential back-off

- Even with random delays, collisions may occur

- Especially likely with busy segments

- Computers double delay with each subsequent collision

- Reduces likelihood of sequence of collisions

Wireless LAN

- Use radio signals at 900MHz

- Data rate of 2Mbps

- Shared medium - radio instead of coax

Limited connectivity with wireless

- In contrast with wired LAN, not all participants may be able to reach each other

  - Low signal strength

  - Propagation blocked by walls, etc.

- Can't depend on CD; not all participants may hear

CSMA/CA

- Wireless uses collision avoidance rather than collision detection

  - Transmitting computer sends very short message to receiver

  - Receiver responds with short message reserving slot for transmitter

- Response from receiver is broadcast so all potential transmitters receive reservation

Collisions

- Receiver may receive simultaneous requests

  - Results in collision at receiver

  - Both requests are lost

  - Neither transmitter receives reservation; both use back-off and retry

- Receiver may receive closely spaced requests

  - Selects one

  - Selected transmitter sends message

  - Transmitter not selected uses back-off and retries

LocalTalk

- LAN technology that uses bus topology

- Interface included with all Macintosh computers

- Relatively low speed - 230.4Kbps

- Low cost (``free'' with a Macintosh); easy to install and connect

- Uses CSMA/CD

Token ring

- Many LAN technologies that use ring topology use token passing for synchronized access to the ring

- Ring itself is treated as a single, shared communication medium

- Bits pass from transmitter, past other computers and are copied by destination

- Hardware must be designed to pass token even if attached computer is powered down

Transmission around a token ring

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap07/chap07_31.html

# Shockwave

Using the token

- When a computer wants to transmit, it waits for the token

- After transmission, computer transmits token on ring

- Next computer ready to transmit receives token and then transmits

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap07/chap07_32.html

# Shockwave

Token and synchronization

- Because there is only one token, only one computer will transmit at a time

  - Token is short, reserved frame that cannot appear in data

  - Hardware must regenerate token if lost

- Token gives computer permission to send one frame

  - If all ready to transmit, enforces ``round-robin'' access

  - If none ready to transmit, token circulates around ring

IBM token ring

- Very widely used

- Originally 4mbps, now 16Mbps

- Uses special connector cable between computer and ring interface

FDDI

- Fiber Distributed Data Interconnect (FDDI) is another ring technology

  - Uses fiber optics between stations

  - Transmits data at 100Mbps

- Uses pairs of fibers to form two concentric rings

FDDI and reliability

- FDDI uses counter-rotating rings in which data flows in opposite directions

- In case of fiber or station failure, remaining stations loop back and reroute data through spare ring

- All stations automatically configure loop back by monitoring data ring

ATM - Star network

- Asynchronous Transfer Mode technology consists of electronic packet switches to which computers can connect

- ATM switches form hub into which computers connect in a star topology

- Computers get point-to-point connections - data from transmitter is routed directly through hub switches to destination

ATM details

- Transmits data at over 100Mbps

- Uses fiber optics to connect computer to switch

- Each connection includes two fibers

ATM switches

Summary

- LAN technologies use shared communication media to interconnect multiple computers over short distances

- Transmitting computer has exclusive use of communication medium; computers must synchronize transmission and share available capacity

- LAN topologies:

  - Star

  - Ring

  - Bus

- LAN technologies

  - Ethernet

  - Wireless

  - LocalTalk

  - IBM Token Ring

  - FDDI

  - ATM

Chapter 8 - Hardware Addressing and Frame Type Identification

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Specifying a destination |
| 3 | Hardware addressing |
| 4 | LAN hardware and packet filtering |
| 5 | LAN hardware and packet filtering |
| 6 | Format of hardware addresses |
| 7 | Assigning hardware addresses |
| 8 | Broadcasting |
| 9 | Identifying packet contents |
| 10 | Headers and frame formats |
| 11 | Example frame format |
| 12 | Ethernet fields |
| 13 | Frames without type fields |
| 14 | Encoding the data type |
| 15 | IEEE 802.2 LLC |
| 16 | Unknown types |
| 17 | Network analyzers |
| 18 | Operation of a network analyzer |
| 19 | Filtering incoming frames |
| 20 | Summary |

Introduction

- Previous chapter on LAN technology described techniques for providing connectivity between computers

- Need to devise technique for delivering message through LAN medium to single, specific destination computer

- Sending computer uses a hardware address to identify the intended destination of a frame

- Sending computer also identifies type of data carried in the frame

Specifying a destination

- Data sent across a shared network reaches all attached stations - for all LAN topologies

- Interface hardware detects delivery of frame and extracts frame from medium

- But ... most applications want data to be delivered to one specific application on another computer - not all computers

Hardware addressing

- Most network technologies have a hardware addressing scheme that identifies stations on the network

- Each station is assigned a numeric hardware address or physical address

- Sender includes hardware address in each transmitted frame

- Only station identified in frame receives copy of frame

- Most LAN technologies include sender's hardware address in frame, too

LAN hardware and packet filtering

- A little detail about organization of LAN hardware and computer:

LAN hardware and packet filtering

- LAN interface handles all details of frame transmission and reception

  - Adds hardware addresses, error detection codes, etc. to outgoing frames

  - May use DMA to copy frame data directly from main memory

  - Obeys access rules (e.g., CSMA/CD) when transmitting

  - Checks error detection codes on incoming frames

  - May use DMA to copy data directly into main memory

  - Checks destination address on incoming frames

- If destination address on incoming frame matches the local station's address, a copy of the frame is passed to the attached computer

- Frames not addressed to the local computer are ignored and don't affect the local computer in any way

Format of hardware addresses

- Numeric value

- Size selected for specific network technology

- Length is one to six bytes

Assigning hardware addresses

- Hardware addresses must be unique on a LAN

- How can those addresses be assigned and who is responsible for uniqueness?

| Static: | Hardware manufacturer assigns permanent address to each interface | Manufacturer must ensure every interface has a unique address |
| --- | --- | --- |
| Dynamic: | Address can be set by end user, either through switches or jumpers on the interface or through software | System administrators must coordinate to avoid conflict |
| Automatic: | Interface automatically assigns hardware address each time it is powered up | Automatic scheme must be reliable to prevent conflicts |

Broadcasting

- Some applications want to broadcast messages to all stations on the LAN

- Shared communication channel can make broadcast efficient - message is delivered to all stations

- Special broadcast address used to identify broadcast messages, which are captured by all stations

Identifying packet contents

- Destination must get some clue about how to interpret frame data

- Can use:

  - Explicit frame type - identifying value included with frame describes type of included data

  - Implicit frame type - receiver must infer type from frame data

Headers and frame formats

- LAN technology standards define frame format for each technology

- All contemporary standards use the following general format:

- Frame header has address and other identifying information

- Information typically in fields with fixed size and location

- Data area may vary in size

Example frame format

- Ethernet frame format:

- Details:

| Field | Purpose |
| --- | --- |
| Preamble | Receiver synchronization |
| Dest. addr. | Identifies intended receiver |
| Source addr. | Hardware address of sender |
| Frame type | Type of data carried in frame |
| Data | Frame payload |
| CRC | 32-bit CRC code |

Ethernet fields

- Preamble and CRC often not shown

- Destination address of all 1s is the broadcast address

- Special values are reserved for frame type field:

Frames without type fields

- Some LAN technologies do not include a type field

- Sender and receiver can agree on interpretation:

  - Agree on a single data format and use only that format

    - Limits LAN to one type of data

    - All computers on LAN must use one format

  - Agree to encode the data format in the first few bytes of the data field

Encoding the data type

- Illustration of using data area to encode data type:

- To ensure interoperability, format of encoding area must be universally agreed upon

- Format typically set by standards body

IEEE 802.2 LLC

- IEEE 802.2 standard includes Logical Link Control (LLC) SubNetwork Attachment Point (SNAP) header

- SNAP/LLC format widely used; e.g., by Ethernet

- LLC portion indicates SNAP field to follow

- OUI (Organizationally Unique Identifier) identifies Ethernet specification organization

- TYPE field interpreted as in Ethernet (in this case, IP)

Unknown types

- For either encoding format, some computers may not be prepared to accept frames of some types

  - Protocol type not installed

  - Newly defined type

- Receiving computer examines type field and discards any frames with unknown type

Network analyzers

- A network analyzer or network monitor or ``network sniffer'' is used to examine the performance of or debug a network

- Can report statistics such as capacity utilization, distribution of frame size, collision rate or token circulation time

- Can record and display specific frames, to understand and debug packet transmissions and exchanges

Operation of a network analyzer

- Basic idea is a computer with a network interface that receives all frames

- Sometimes called promiscuous mode

- Many desktop computers have interface that can be configured for promiscuous mode

  - Combined with software, computer can examine any frame on LAN

  - Communication across a LAN is not guaranteed to be private!

- Computer receives and displays (but does not respond to) frames on the LAN

Filtering incoming frames

- Analyzer can be configured to filter and process frames

  - Count frames of a specific type or size

  - Display only frames from or to specific computers

  - In general, can be configured to match value of any field and capture only those frames meeting the filter specification

- Analyzer can display real-time performance by computing running totals over specific time periods

Summary

- LAN technologies use hardware addresses to identify destination for frames sent across shared communication channel

- Each LAN technology defines its own hardware format

- Addresses may be statically assigned, configurable or automatically assigned

- Each station must have a unique address on the LAN segment

- Frames include a header with fields for destination, source and other information such as frame type

- Frame type defines how to interpret frame data

- Network analyzer can receive all frames and display statistics or aid in debugging problems

Chapter 9 - LAN Wiring, Physical Topology and Interface Hardware

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Speeds of LANs and computers |
| 3 | Network interface hardware |
| 4 | I/O interfaces |
| 5 | Network connector |
| 6 | NICs and network hardware |
| 7 | NIC and CPU processing |
| 8 | Connection between NIC and physical network |
| 9 | Thick Ethernet wiring |
| 10 | Thick Ethernet example |
| 11 | Connection multiplexing |
| 12 | Thin Ethernet wiring |
| 13 | Thin Ethernet wiring (continued) |
| 14 | Thin Ethernet wiring (continued) |
| 15 | 10Base-T |
| 16 | Hubs |
| 17 | Protocol software and Ethernet wiring |
| 18 | Comparison of wiring schemes |
| 19 | Comparison of wiring schemes (continued) |
| 20 | Topologies and network technologies |
| 21 | Other technologies |
| 22 | Technology translation |
| 23 | Summary |

Introduction

- Interface cards

  - Why a separate card

  - How to connect the interface to the computer

  - What is a ``transceiver''?

- LAN wiring schemes

- Logical and physical topology

Speeds of LANs and computers

- LAN data transmission speeds are typically ``fast'' relative to CPU speeds

- 100MHz CPU could execute only one instruction for each bit on a 100Mhz Ethernet

- LAN speeds are defined independent of any specific processor speeds

  - Allows for mix of attached systems

  - New computers can be attached without affecting LAN speeds

Network interface hardware

- CPU can't process data at network speeds

- Computer systems use special purpose hardware for network connection

  - Typically a separate card in the backplane

  - Network adapter card or network interface card (NIC)

- Connector at back of computer then accepts cable to physical network

I/O interfaces

Network connector

NICs and network hardware

- NIC is built for one kind of physical network

  - Ethernet interface can't be used with token ring

  - ATM interface can't be used with FDDI

- Some NICs can be used with different, similar hardware

  - Thick, thin and 10Base-T Ethernet

  - 10Mbps and 100Mbps Ethernet

NIC and CPU processing

- NIC contains sufficient hardware to process data independent of system CPU

  - Some NICs contain separate microprocessor

  - Includes analog circuitry, interface to system bus, buffering and processing

- Looks like any other I/O device to system CPU

  - System CPU forms message request

  - Sends instructions to NIC to transmit data

  - Receives interrupt on arrival of incoming data

Connection between NIC and physical network

- Two alternatives:

  - NIC contains all circuitry and connects directly to network medium

  - Cable from NIC connects to additional circuitry that then attaches to the network medium

- Thin Ethernet vs. 10Base-T

- Both are Ethernet; network technology not limited to one style of connection

Thick Ethernet wiring

- Uses thick coax cable

- AUI cable (or transceiver or drop cable connects from NIC to transceiver

- AUI cable carries digital signal from NIC to transceiver

- Transceiver generates analog signal on coax

- Wires in AUI cable carry digital signals, power and other control signals

Thick Ethernet example

- Thick Ethernet also requires termination to avoid signal reflectance

Connection multiplexing

- In some circumstances, transceivers may be inconvenient; e.g., workstations in a lab

- Connection multiplexor connects multiple computers to a single transceiver

  - Each computer's AUI cable connects to connection multiplexor

  - One AUI from multiplexor to Ethernet coax

- Connection multiplexor completely invisible to attached computers

Thin Ethernet wiring

- Uses thin coax that is cheaper and easier to install than thick Ethernet coax

- Transceiver electronics built into NIC; NIC connects directly to network medium

- Coax cable uses BNC connector

Thin Ethernet wiring (continued)

- Coax runs directly to back of each connected computer

- T connector attaches directly to NIC

Thin Ethernet wiring (continued)

- Useful when many computers are located close to each other

- May be unreliable - any disconnection disrupts entire net

10Base-T

- Variously called 10Base-T, twisted pair or TP Ethernet

- Replaces AUI cable with twisted pair cable

- Replaces thick coax with hub

Hubs

- Extension of connection multiplexing concept

- Sometimes called ``Ethernet-in-a-box''

- Effectively a very short Ethernet with very long AUI cables

- Can be connected into larger Ethernets

Protocol software and Ethernet wiring

- All wiring technologies use identical Ethernet specification

  - Same frame format

  - Same CSMA/CD algorithms

- Can mix different technologies in one Ethernet

- NICs can provide all three connection technologies

- Protocol software can't differentiate among wiring technologies

Comparison of wiring schemes

- Separate transceiver allows computer to be powered off or disconnected from network without disrupting other communication

- Transceiver may be located in an inconvenient place

- Finding malfunctioning transceiver can be hard

- Thin coax takes minimum of cable

- Disconnecting one computer (or one loose connection) can disrupt entire network

- Hub wiring centralizes electronics and connections, making management easier

- Bottom line - 10Base-T most popular because of cost

Comparison of wiring schemes (continued)

Topologies and network technologies

- 10Base-T network topology is a bus; wiring topology is a star

- Token ring network topology is a ring; wiring topology is a star

- Remember to distinguish between logical and physical topologies

Other technologies

- AppleTalk uses bus wiring with coax cable between transceivers

- AppleTalk can also use hub technology or spare wires in 4-wire phone cable

Technology translation

- Adapters can translate between some network technologies

  - Ethernet AUI-to-thinnet

  - Ethernet AUI-to-10Base-T adapters

- AppleTalk to phone wire

Summary

- Network interface card (NIC) connects computer system to network

  - NIC operates independently; is fast enough to keep up with network

  - Typically uses interrupts to interact with CPU

- Many physical wiring schemes are available for logical network topology

- 10Base-T is a logical bus and a physical star

Chapter 10 Extending LANs: Fiber Modems, Repeaters, Bridges and Switches

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | LAN design for distance |
| 3 | LAN extensions |
| 4 | Fiber optic extensions |
| 5 | Repeaters |
| 6 | Ethernet repeaters |
| 7 | Limits on repeaters |
| 8 | Repeater architecture |
| 9 | Characteristics of repeaters |
| 10 | Bridges |
| 11 | Bridged LAN segments |
| 12 | Characteristics of bridges |
| 13 | Filtering bridges |
| 14 | Frame filtering |
| 15 | How does bridge set up table? |
| 16 | Filtering example |
| 17 | Startup behavior of filtering bridges |
| 18 | Designing with filtering bridges |
| 19 | Bridging between buildings |
| 20 | Bridging across longer distances |
| 21 | Bridges and cycles |
| 22 | Cycles of bridges |
| 23 | Eliminating broadcast cycles |
| 24 | Switching |
| 25 | Switches and hubs |
| 26 | Summary |

Introduction

- LAN technologies are designed with constraints of speed, distance and costs

- Typical LAN technology can span, at most, a few hundred meters

- How can a network be extended to cover longer distances; e.g., the Bucknell campus?

LAN design for distance

- Many LANs use shared medium - Ethernet, token ring

- Length of medium affects fair, shared access to medium

  - CSMA/CD - delay between frames, minimum frame length

  - Token passing - circulation time for token

- Length of medium affects strength of electrical signals and noise immunity

LAN extensions

- Several techniques extend diameter of LAN medium

- Most techniques use additional hardware

- LAN signals relayed between LAN segments

- Resulting mixed technology stays within original engineering constraints while spanning greater distance

Fiber optic extensions

- Can extend connection to a computer using fiber optic cable

- Insert fiber modems and fiber optic cable into AUI cable

- Fiber modems:

  - Convert AUI signals to digital signal

  - Transmit digital signals via fiber optic cable to other modem

- Most often used to connect two LANs - typically through a bridge - different buildings

Repeaters

- May want to extend LAN medium

  - Ethernet - timing constraints allow longer medium

  - Signal strength constraints limit length

- Repeater - bidirectional, analog amplifier that retransmits analog signals

- One repeater can effectively double the length of an LAN segment

Ethernet repeaters

- Simply copy signals between segments

  - Do not understand frame formats

  - Do not have hardware addresses

- Any Ethernet segment is limited to 500 meters

- Repeater can double to 1,000 meters

Limits on repeaters

- Can't extend Ethernet with repeaters indefinitely

- CSMA/CD requires low delay; if medium is too long, CSMA/CD won't work

- Ethernet standard includes limit of 4 repeaters between any two Ethernet stations

Repeater architecture

- With four repeaters, can extend Ethernet through a building

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap10/chap10_8.html

### Shockwave

- FOIRL - Fiber Optic Intra-Repeater Link - can be used to connect bridges

Characteristics of repeaters

- Very easy to use - just plug in

- Repeaters simply re-transmit analog signals

  - Collisions affect entire network

  - Transient problems - noise - propagates throughout network

Bridges

- Also connect two LAN segments

- Retransmits frames from one segment on other segment(s)

- Handles complete frame

  - Uses NIC like any other station

  - Performs some processing on frame

- Invisible to other attached computers

Bridged LAN segments

Characteristics of bridges

- Relatively easy to use - just plug in

- Isolate collisions, noise

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap10/chap10_12.html

#### Shockwave

Filtering bridges

- Bridges can do additional processing

  - Don't forward collisions, noise

  - Only forward frames where necessary

- Bridge performs frame filtering and forwards frames along LAN segments to destination

  - Learns location of stations by watching frames

  - Forwards all broadcast and multicast packets

Frame filtering

- Bridge checks destination of each incoming frame

- Looks up destination in list of known stations

  - Forwards frame to next interface on path to destination

  - Doesn't forward frame if destination on LAN segment from which frame was received

How does bridge set up table?

- Bridge examines source address in each frame

- Adds entry to list for LAN segment from which frame was received

- Must forward any frame whose destination is not in the list on every interface

Filtering example

Startup behavior of filtering bridges

- Initially, the forwarding tables in all bridges are empty

- First frame from each station on LAN is forwarded to all LAN segments

- After all stations have been identified, frames are only forwarded as needed

- May result in burst of traffic after, e.g., power failure

Designing with filtering bridges

- Filtering bridge allows concurrent use of different LAN segments if traffic is local

- U and V can exchange frames at the same time X and Y exchange frames

- Designers identify patterns of local communication and isolate groups of communicating computers with bridges

Bridging between buildings

- Similar to extending AUI with fiber modems

- Can put bridge in one building with long connection to LAN segment in different building

- Avoids extended AUI connection for each computer in remote building

Bridging across longer distances

- Can use leased line, microwave, laser or satellite to connect two bridges and LAN segments

- Using two bridges instead of one:

  - Filters at both ends, reducing traffic across slow link

  - Provides buffering at both ends, matching dissimilar transmission speeds

Bridges and cycles

- Can use multiple bridges to interconnect many LAN segments

- Station of segment c sends frames to station on segment g through B2, B1, B3 and B6

- Broadcasts are forwarded through all bridges

- Suppose another bridge connects g and f?

Cycles of bridges

- A circular path through bridged networks is called a cycle

- Adding B4 creates a cycle

Eliminating broadcast cycles

- Bridges must cooperate to broadcast frames exactly once on each segment

- Solution from graph theory - spanning tree - used to determine which bridges will forward broadcasts

- As each bridge joins the network, it communicates with other bridges on special hardware (typically multicast) address

  - Learns network topology

  - Performs spanning tree computation

  - Determines if bridge will form a cycle

Switching

- Effectively a separate LAN segment for each port

- Similar to hub - hub shares single segment among all ports

- With switching, multiple stations can transmit simultaneously

- Provides much higher aggregate bandwidth

Switches and hubs

- Switches are more expensive per port

- May make more sense economically to use hubs for some stations and switches for others

Summary

- Optical fiber and modems can be used to extend AUI for single station

- Repeater acts as amplifier and retransmits analog signals

- Bridge accepts entire incoming frame and retransmits

  - Doesn't forward collisions

  - Avoids collisions on destination segments

- Filtering bridge forwards frames only as needed

  - Allows simultaneous use of LAN segments for local transmission

  - Forwards all broadcast and multicast packets

- Switches provide full LAN speed to each port by simulating separate LAN segments

Chapter 11 Long-Distance Digital Connection Technologies

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Digital telephony |
| 3 | Digitizing voice |
| 4 | Example |
| 5 | Sampling parameters |
| 6 | Synchronous communication |
| 7 | Using digital telephony for data delivery |
| 8 | Conversion for digital circuits |
| 9 | Using DSU/CSU |
| 10 | Telephone standards |
| 11 | Intermediate capacity |
| 12 | Higher capacity circuits |
| 13 | About the terminology |
| 14 | SONET |
| 15 | Getting to your home |
| 16 | ISDN |
| 17 | DSL |
| 18 | ADSL technology |
| 19 | Adaptive transmission |
| 20 | Other DSL technologies |
| 21 | Cable modem technologies |
| 22 | Features of cable modems |
| 23 | Upstream communication |
| 24 | Alternatives |
| 25 | Summary |

Introduction

- Previous technologies cover "short" distances

- Can extend over short distances

- Need to cover longer distances - e.g., Bucknell to New York

- Will call this technology WAN - Wide Area Network

- Two categories:

  - Long distance between networks

  - "Local loop"

Digital telephony

- Telephone system spans long distances

- Digital telephony improved long distance service:

  - Better quality

  - More connections in wire

Digitizing voice

- Problem: encode analog audio signal as digital data

- Solution:

  - Sample audio signal at periodic intervals

  - Convert to digital using A-to-D converter

  - Send data over wire

  - Reconvert to audio using D-to-A converter

Example

Sampling parameters

- Want to carry signals up to 4000Hz

- Select sample rate of 8000Hz

- Each sample is in range 0-255 (8 bits)

- Standard called Pulse Code Modulation (PCS

Synchronous communication

- Converting back to audio requires data be available "on time"

- Digital telephony systems use clocking for synchronous data delivery

- Samples not delayed as traffic increases

Using digital telephony for data delivery

- So, digital telephony can handle synchronous data delivery

- Can we use that for data delivery?

- Ethernet frame != 8-bit PCM synchronous

- Need to convert formats...

Conversion for digital circuits

- To use digital telephony for data delivery:

  - Lease point-to-point digital circuit between sites

  - Convert between local and PCM formats at each end

- Use a Data Service Unit/Channel Service Unit (DSU/CSU) at each end

  - CSU - manages control functions

  - DSU - converts data

Using DSU/CSU

Telephone standards

- Several standards exist for data transmission rates

- Called T-series standards

| Name | Bit Rate | Voice Circuits |
| --- | --- | --- |
| - | 0.064 Mbps | 1 |
| T1 | 1.544 Mbps | 24 |
| T2 | 6.312 Mbps | 96 |
| T3 | 44.736 Mbps | 672 |

Intermediate capacity

- Price does not go up linearly with speed

- $$ for T3 < $$ for 28 * T1
...however, if all you need is 9 Mbps, $$ for T3 > $$ for 6 * T1

- Solution: combine multiple T1 lines with inverse multiplexor

Higher capacity circuits

| Standard name | Optical name | Bit rate | Voice circuits |
| --- | --- | --- | --- |
| STS-1 | OC-1 | 51.840 Mbps | 810 |
| STS-3 | OC-3 | 155.520 Mbps | 2,430 |
| STS-12 | OC-12 | 622.080 Mbps | 9,720 |
| STS-24 | OC-24 | 1,244.160 Mbps | 19,440 |
| STS-48 | OC-48 | 2,488.320 Mbps | 38,880 |

About the terminology

- T-standards define underlying bit rate, Digital Signal Level standards (DS standards) define:

  - how to multiplex calls

  - effective bit rates

  - T1 line transmit data at DS-1 rate

- Synchronous Transport Signal (STS) standards define high speed connections over copper, Optical Carrier (OC) standard are for fiber

- C suffix indicates concatenated:

  - OC-3 == three OC-1 circuits at 51.84 Mbps

  - OC-3C == one 155.52 Mbps circuit

SONET

- Synchronous Optical Network (SONET) defines how to use high-speed connections

  - Framing: STS-1 uses 810 bytes per frame

  - Encoding: Each sample travels as one octet in payload

Getting to your home

- Local loop describes connection from telephone office to your home

- Sometimes called POTS (Plain Old Telephone Service)

- Legacy infrastructure is copper

- Other available connections include cable TV, wireless, electric power

ISDN

- Provides digital service (like T-series) on existing local loop copper

- Three separate circuits, or channels

  - Two B channels, 64 Kbps each; == 2 voice circuits

  - One D channel, 16 Kbps; control

- Often written as 2B+D; called Basic Rate Interface (BRI)

- Slow to catch on

  - Expensive

  - Charged by time used

  - (Almost) equaled by analog modems

DSL

- DSL (Digital Subscriber Line) is a family of technologies

  - Sometimes called xDSL

  - Provides high-speed digital service over existing local loop

- One common form is ADSL (Asymmetric DSL)

  - Higher speed into home than out of home

  - More bits flow in ("downstream") than out ("upstream")

- ADSL maximum speeds:

  - 6.144 Mbps downstream

  - 640 Kbps upstream

ADSL technology

- Uses existing local loop copper

- Takes advantage of higher frequencies on most local loops

- Can be used simultaneously for POTS

Adaptive transmission

- Individual local loops have different transmission characteristics

  - Different maximum frequencies

  - Different interference frequencies

- ADSL uses FDM

  - 286 frequencies

    - 255 downstream

    - 31 upstream

    - 2 control

- Each frequency carries data independently

  - All frequencies out of audio range

  - Bit rate adapts to quality in each frequency

Other DSL technologies

- SDSL (Symmetric DSL) provides divides frequencies evenly

- HDSL (High-rate DSL) provides DS1 bit rate both directions

  - Short distances

  - Four wires

- VDSL(Very high bit rate DSL) provides up to 52 Mbps

  - Very short distance

  - Requires Optical Network Unit (ONU) as a relay

Cable modem technologies

- Cable TV already brings high bandwidth coax into your house

- Cable modems encode and decode data from cable TV coax

  - One in cable TV center connects to network

  - One in home connects to computer

Features of cable modems

- Bandwidth multiplexed among all users

- Multiple access medium; your neighbor can see your data!

- Not all cable TV coax plants are bi-directional

Upstream communication

- Cable TV is one direction

  - Signal broadcast at central location

  - Amplifiers boost signal through network

- Amplifiers are unidirectional!

- Solutions:

  - Retrofit bi-directional amplifiers

  - Alternate upstream path - e.g., dialup

Alternatives

- Satellite

- "Fiber to the curb'

Summary

- WAN links between sites use digital telephony

  - Based on digitized voice service

  - Several standard rates

  - Requires conversion vis DSU/CSU

- Local loop technologies

  - ISDN

  - xDSL

  - Cable modem

  - Satellite

  - Fiber to the curb

Chapter 12 - WAN Technologies and Routing

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Characterizations of networks |
| 3 | Differences between LAN and WAN |
| 4 | Packet switches |
| 5 | Connections to packet switches |
| 6 | Packet switches as building blocks |
| 7 | Store and forward |
| 8 | Store and forward example |
| 9 | Physical addressing in a WAN |
| 10 | Next-hop forwarding |
| 11 | Choosing next hop |
| 12 | Source independence |
| 13 | Hierarchical address and routing |
| 14 | WAN architecture and capacity |
| 15 | Routing in a WAN |
| 16 | Modeling a WAN |
| 17 | Route computation with a graph |
| 18 | Redundant routing information |
| 19 | Default routes |
| 20 | Building routing tables |
| 21 | Computation of shortest path in a graph |
| 22 | Weighted graph |
| 23 | Synopsis of Djikstra's algorithm |
| 24 | Distance metrics |
| 25 | Dynamic route computation |
| 26 | Distributed route computation |
| 27 | Vector-distance algorithm |
| 28 | Vector-distance algorithm (continued) |
| 29 | Link-state routing |
| 30 | Comparison |
| 31 | Examples of WAN technology |
| 32 | Summary |

Introduction

- LANs can be extended using techniques in previous chapter

- Can not be extended arbitrarily far or to handle arbitrarily many computers

  - Distance limitations even with extensions

  - Broadcast a problem

- Need other technologies for larger networks

Characterizations of networks

- Local Area Network (WAN) - single building

- Metropolitan Area Network (MAN) - single city

- Wide Area network (WAN) - country, continent, planet

Differences between LAN and WAN

- Satellite bridge can extend LAN across large distances

- Still cannot accommodate arbitrarily many computers

- WAN must be scalable to long distances and many computers

Packet switches

- To span long distances or many computers, network must replace shared medium with packet switches

  - Each switch moves an entire packet from one connection to another

  - A small computer with network interfaces, memory and program dedicated to packet switching function

Connections to packet switches

- Packets switches may connect to computers and to other packet switches

- Typically high speed connections to other packets switches, lower speed to computers

- Technology details depend on desired speed

Packet switches as building blocks

- Packet switches can be linked together to form WANs

- WANs need not be symmetric or have regular connections

- Each switch may connect to one or more other switches and one or more computers

Store and forward

- Data delivery from one computer to another is accomplished through store-and-forward technology

  - Packet switch stores incoming packet

  - ... and forwards the packet to another switch or computer

- Packet switch has internal memory

  - Can hold packet if outgoing connection is busy

  - Packets for each connection held on queue

Store and forward example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap12/chap12_8.html

Shockwave

Physical addressing in a WAN

- Similar to LAN

  - Data transmitted in packets (equivalent to frames)

  - Each packet has format with header

  - Packet header includes destination and source addresses

- Many WANs use hierarchical addressing for efficiency

  - One part of address identifies destination switch

  - Other part of address identifies port on switch

Next-hop forwarding

- Packet switch must choose outgoing connection for forwarding

  - If destination is local computer, packet switch delivers computer port

  - If destination is attached another switch, this packet switch forwards to next hop through connection to another switch

- Choice based on destination address in packet

Choosing next hop

- Packet switch doesn't keep complete information about all possible destination

- Just keeps next hop

- So, for each packet, packet switch looks up destination in table and forwards through connection to next hop

Source independence

- Next hop to destination does not depend on source of packet

- Called source independence

- Allows fast, efficient routing

- Packet switch need not have complete information, just next hop

  - Reduces total information

  - Increases dynamic robustness - network can continue to function even if topology changes without notifying entire network

Hierarchical address and routing

- Process of forwarding is called routing

- Information is kept in routing table

- Note that many entries have same next hop

- In particular, all destinations on same switch have same next hop

- Thus, routing table can be collapsed:

WAN architecture and capacity

- More computers == more traffic

- Can add capacity to WAN by adding more links and packet switches

- Packet switches need not have computers attached

- Interior switch - no attached computers

- Exterior switch - attached computers

Routing in a WAN

- Both interior and exterior switches:

  - Forward packets

  - Need routing tables

- Must have:

  - Universal routing - next hop for each possible destination

  - Optimal routes - next hop in table must be on shortest path to destination

Modeling a WAN

- Use a graph:

  - Nodes model switches

  - Edges model direct connections between switches

- Captures essence of network, ignoring attached computers

Route computation with a graph

- Can represent routing table with edges:

- Graph algorithms can be applied to find routes

Redundant routing information

- Notice duplication of information in routing table for node 1:

- Switch 1 has only one outgoing connection; all traffic must traverse that connection

- Extrapolate to Bucknell and Internet

Default routes

- Can collapse routing table entries with a default route

- If destination does not have an explicit routing table entry, use use the default route:

- Use of default route is optional (see node 3)

- Extrapolate to Bucknell

Building routing tables

- How to enter information into routing tables:

  - Manual entry - initialization file

  - Dynamically - through runtime interface

- How to compute routing table information:

  - Static routing - at boot time

  - Dynamic routing - allow automatic updates by a program

- Static is simpler; doesn't accommodate changes to network topology

- Dynamic requires additional protocol(s); can work around network failures

Computation of shortest path in a graph

- Assume graph representation of network at each node

- Use Djikstra's algorithm to compute shortest path from each node to every other node

- Extract next-hop information from resulting path information

- Insert next-hop information into routing tables

Weighted graph

- Djikstra's algorithm can accommodate weights on edges in graph

- Shortest path is then the path with lowest total weight (sum of weights of all edges)

- Shortest path not necessarily fewest edges (or hops)

Synopsis of Djikstra's algorithm

- Keep data structure with list of nodes and weights of paths to those nodes

- Use infinity to represent a node in the set S of nodes for which a path has not yet been computed

- At each iteration, find a node in S, compute the path to that node, and delete the node from S

Synopsis of Djikstra's algorithm

- Keep data structure with list of nodes and weights of paths to those nodes

- Use infinity to represent a node in the set S of nodes for which a path has not yet been computed

- At each iteration, find a node in S, compute the path to that node, and delete the node from S

Distance metrics

- Weights on graph edges reflect "cost" of traversing edge

  - Time

  - Dollars

  - Hop count (weight == 1)

- Resulting shortest path may not have fewest hops

Dynamic route computation

- Network topology may change dynamically

  - Switches may be added

  - Connections may fail

  - Costs for connections may change

- Switches must update routing tables based on topology changes

Distributed route computation

- Pass information about network topology between nodes

- Update information periodically

- Each node recomputes shortest paths and next hops

- Inject changes into routing tables

Vector-distance algorithm

- Local information is next-hop routing table and distance from each switch

- Switches periodically broadcast topology information

- Other switches update routing table based on received information

Vector-distance algorithm (continued)

- In more detail:

- Wait for next update message

- Iterate through entries in message

- If entry has shorter path to destination:

- Insert source as next hop to destination

- Record distance as distance from next hop to destination PLUS

- Distance from this switch to next hop

Link-state routing

- Separates network topology from route computation

- Switches send link-state information about local connections

- Each switch builds own routing tables

  - Uses link-state information to update global topology

  - Runs Djikstra's algorithm

Comparison

- Vector-distance algorithm

  - Very simple to implement

  - May have convergence problems

  - Used in RIP

- Link-state algorithm

  - Much more complex

  - Switches perform independent computations

  - Used in OSPF

Examples of WAN technology

- ARPANET

  - Began in 1960s

  - Funded by Advanced Research Projects Agency, an organization of the US Defense Department

  - Incubator for many of current ideas, algorithms and internet technologies

  - See Where Wizards Stay Up Late

- X.25

  - Early standard for connection-oriented networking

  - From ITU, which was originally CCITT

  - Predates computer connections, used for terminal/timesharing connection

- Frame Relay

  - Telco service for delivering blocks of data

  - Connection-based service; must contract with telco for circuit between two endpoints

  - Typically 56Kbps or 1.5Mbps; can run to 100Mbps

- SMDS - Switched Multi-megabit Data Service

  - Also a Telco service

  - Connectionless service; any SMDS station can send a frame to any other station on the same SMDS "cloud"

  - Typically 1.5-100Mbps

- ATM - Asynchronous Transfer Mode

  - Designed as single technology for voice, video, data, ...

  - Low jitter (variance in delivery time) and high capacity

  - Uses fixed size, small cells - 48 octets data, 5 octets header

  - Can connect multiple ATM switches into a network

Summary

- WAN can span arbitrary distances and interconnect arbitrarily many computers

- Uses packet switches and point-to-point connections

- Packets switches use store-and-forward and routing tables to deliver packets to destination

- WANs use hierarchical addressing

- Graph algorithms can be used to compute routing tables

- Many LAN technologies exist

Chapter 14 - Protocols and Layering

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Why network software? |
| 3 | Why protocols? |
| 4 | One or many protocols? |
| 5 | Protocol suites |
| 6 | Layered protocol design |
| 7 | The ISO 7-layer reference model |
| 8 | The layers in the ISO model |
| 9 | Layered software implementation |
| 10 | Layered software and stacks |
| 11 | Layering principle |
| 12 | Messages and protocol stacks |
| 13 | Commercial stacks |
| 14 | Protocol headers |
| 15 | Control packets |
| 16 | Techniques for reliable network communication |
| 17 | Out-of-order delivery |
| 18 | Duplicate delivery |
| 19 | Lost packets |
| 20 | Retransmission |
| 21 | Replay |
| 22 | Flow control |
| 23 | Stop-and-go flow control |
| 24 | Sliding window |
| 25 | Example of sliding window |
| 26 | Comparison of stop-and-go and sliding window |
| 27 | Transmission times |
| 28 | Network congestion |
| 29 | Aoviding and recovering from network congestion |
| 30 | Art, engineering and protocol design |
| 31 | Summary |

Introduction

- LAN/WAN hardware can't solve all computer communication problems

- Software for LAN and WAN systems is large and complicated

- Layering is a structuring technique to organize networking software design and implementation

Why network software?

- Sending data through raw hardware is awkward and inconvenient - doesn't match programming paradigms well

- Equivalent to accessing files by making calls to disk controller to position read/write head and accessing individual sectors

- May not be able to send data to every destination of interest without other assistance

- Network software provides high-level interface to applications

Why protocols?

- Name is derived from the Greek protokollen, the index to a scroll

- Diplomats use rules, called protocols, as guides to formal interactions

- A network protocol or computer communication protocol is a set of rules that specify the format and meaning of messages exchanged between computers across a network

  - Format is sometimes called syntax

  - Meaning is sometimes called semantics

- Protocols are implemented by protocol software

One or many protocols?

- Computer communication across a network is a very hard problem

- Complexity requires multiple protocols, each of which manages a part of the problem

- May be simple or complex; must all work together

Protocol suites

- A set of related protocols that are designed for compatibility is called a protocol suite

- Protocol suite designers:

  - Analyze communication problem

  - Divide problems into subproblems

  - Design a protocol for each subproblem

- A well-designed protocol suite

  - Is efficient and effective - solves the problem without redundancy and makes best use of network capacity

  - Allows replacement of individual protocols without changes to other protocols

Layered protocol design

- Layering model is a solution to the problem of complexity in network protocols

- Model suggests dividing the network protocol into layers, each of which solves part of the network communication problem

- These layers have several constraints, which ease the design problem

- Network protocol designed to have a protocol or protocols for each layer

The ISO 7-layer reference model

- International Organization for Standards (ISO) defined a 7-layer reference model as a guide to the design of a network protocol suite

- Layers are named and numbered; reference to ``layer n'' often means the nth layer of the ISO 7-layer reference model

The layers in the ISO model

- Caveat - many modern protocols do not exactly fit the ISO model, and the ISO protocol suite is mostly of historic interest

- Concepts are still largely useful and terminology persists

Layer 7: Application

Application-specific protocols such as FTP and SMTP (electronic mail)

Layer 6: Presentation

Common formats for representation of data

Layer 5: Session

Management of sessions such as login to a remote computer

Layer 4: Transport

Reliable delivery of data between computers

Layer 3: Network

Address assignment and data delivery across a physical network

Layer 2: Data Link

Format of data in frames and delivery of frames through network interface

Layer 1: Physical

Basic network hardware - such as RS-232 or Ethernet

Layered software implementation

- Software implemented from layered design has layered organization

- Software modules can be viewed as:

Layered software and stacks

- Related modules from previous figure are called a protocol stack or simply a stack

- Two constraints:

  - The software for each layer depends only on the services of the software provided by lower layers

  - The software at layer n at the destination receives exactly the same protocol message sent by layer n at the sender

- These constraints mean that protocols can be tested independently and can be replaced within a protocol stack

Layering principle

Messages and protocol stacks

- On the sender, each layer:

  - Accepts an outgoing message from the layer above

  - Adds a header and other processing

  - Passes resulting message to next lower layer

- On the receiver, each layer:

  - Receives an incoming message from the layer below

  - Removes the header for that layer and performs other processing

  - Passes the resulting message to the next higher layer

Commercial stacks

Protocol headers

- The software at each layer communicates with the corresponding layer through information stored in headers

- Each layer adds its header to the front of the message from the next higher layer

- Headers are nested at the front of the message as the message traverses the network

Control packets

- Protocol layers often need to communicate directly without exchanging data

  - Acknowledge incoming data

  - Request next data packet

- Layers use control packets

  - Generated by layer n on sender

  - Interpreted by layer n on receiver

  - Transmitted like any other packet by layers n-1 and below

Techniques for reliable network communication

- Model - reliable delivery of a block of data from one computer to another

  - Data values unchanged

  - Data in order

  - No missing data

  - No duplicated data

- Example - parity bit, checksum and CRC used to ensure data is unchanged

Out-of-order delivery

- Packets may be delivered out of order - especially in systems that include multiple networks

- Out of order delivery can be detected and corrected through sequencing

  - Sender attaches sequence number to each outgoing packet

  - Received uses sequence numbers to put packets in order and detect missing packets

Duplicate delivery

- Packets may be duplicated during transmission

- Sequencing can be used to...

  - Detect duplicate packets with duplicated sequence numbers

  - Discard those duplicate packets

Lost packets

- Perhaps the most widespread problem is lost packets

- Any error - bit error, incorrect length - causes receiver to discard packet

- Tough problem to solve - how does the receiver decide when a packet has been lost?

Retransmission

- Protocols use positive acknowledgment with retransmission to detect and correct lost packets

  - Receiver sends short message acknowledging receipt of packets

  - Sender infers lost packets from missing acknowledgments

  - Sender retransmits lost packets

- Sender sets timer for each outgoing packet

  - Saves copy of packet

  - If timer expires before acknowledgment is received, sender can retransmit saved copy

- Protocols define upper bound on retransmission to detect unrecoverable network failure

Replay

- Sufficiently delayed packets may be inserted into later sessions

- Suppose two computers exchange data with packets numbered 1 to 5

- Packet 4 encounters an extraordinary delay; computers use retransmission to deliver valid copy of packet 4

- Two computers exchange data later on with packets numbered 1 to 10

- Initial `packet 4' can arrive during second session, so that the data from that old packet (rather than the current `packet 4' is inserted into the data

- Protocols attach session number to each packet in a protocol session to differentiate packets from different sessions

Flow control

- Data overrun can occur when sender transmits data faster than receiver can process incoming data

- Protocols use flow control mechanisms through which receiver can control rate of data transmission

  - Stop-and-go

  - Sliding window

Stop-and-go flow control

- Receiver sends small control packet when it is ready for next packet

- Sender waits for control packet before sending next packet

- Can be very inefficient of network bandwidth if delivery time is large

Sliding window

- Allows sender to transmit multiple packets before receiving an acknowledgment

- Number of packets that can be sent is defined by the protocol and called the window

- As acknowledgments arrive from the receiver, the window is moved along the data packets; hence ``sliding window''

Example of sliding window

Comparison of stop-and-go and sliding window

Transmission times

- For stop-and-go, each packet takes 2L time to deliver (where L is the latency, or network delivery time)

- Sliding window can improve by number of packets in window:

Tw = Tg * W

(Tw is sliding window throughput, Tg is stop-and-go throughput

- Transmission time also limited by network transmission rate:

Tw = min(B, Tg * W)

(B is maximum network bandwidth)

Network congestion

- Network congestion arises in network systems that include multiple links

- If input to some link exceeds maximum bandwidth, packets will queue up at connection to that link

- Eventually, packets will be discarded and packets will be retransmitted

- Ultimately, network will experience congestion collapse

- Problem related to, but not identical to, data overrun

Avoiding and recovering from network congestion

- Protocols attempt to avoid congestion and recover from network collapse by monitoring the state of the network and taking appropriate action

- Can use two techniques:

  - Notification from packet switches

  - Infer congestion from packet loss

- Packet loss can be used to detect congestion because modern networks are reliable and rarely lose packets through hardware failure

- Sender can infer congestion from packet loss through missing acknowledgments

- Rate or percentage of lost packets can be used to gauge degree of congestion

Art, engineering and protocol design

- Protocol design mixes engineering and art

  - There are well-known techniques for solving specific problems

  - Those techniques interact in subtle ways

  - Resulting protocol suite must account for interaction

- Efficiency, effectiveness, economy must all be balanced

Summary

- Layering is a technique for guiding protocol design and implementation

- Protocols are grouped together into related protocol suites

- A collection of layered protocols is called a protocol stack

- Protocols use a variety of techniques for reliable delivery of data

Chapter 15 - Internetworking

| Section | Title |
| --- | --- |
| 1 | Motivation |
| 2 | Universal service |
| 3 | Internetworking |
| 4 | Routers |
| 5 | Internet architecture |
| 6 | Routers in an organization |
| 7 | A virtual network |
| 8 | A protocol suite for internetworking |
| 9 | Internetworking protocols |
| 10 | TCP/IP layering |
| 11 | Hosts, routers and protocol layers |
| 12 | Summary |

Motivation

- There are many different LAN and WAN technologies

- In real world, computers are connected by many different technologies

- Any system that spans a large organization must accommodate multiple technologies

Universal service

- Telephones are useful because any telephone can reach any other telephone

- Universal service among computers greatly increases the usefulness of each computer

- Providing universal service requires interconnecting networks employing different technologies

Internetworking

- Internetworking is a scheme for interconnecting multiple networks of dissimilar technologies

- Uses both hardware and software

  - Extra hardware positioned between networks

  - Software on each attached computer

- System of interconnected networks is called an internetwork or an internet

Routers

- A router is a hardware component used to interconnect networks

- A router has interfaces on multiple networks

- Networks can use different technologies

- Router forwards packets between networks

- Transforms packets as necessary to meet standards for each network

Internet architecture

- An internetwork is composed of arbitrarily many networks interconnected by routers

- Routers can have more than two interfaces

Routers in an organization

- Would be possible to interconnect all networks in an organization with a single router

- Most organizations use multiple routers

  - Each router has finite capacity; single router would have to handle all traffic across entire organization

  - Because internetworking technology can automatically route around failed components, using multiple routers increases reliability

A virtual network

- Internetworking software builds a single, seamless virtual network out of multiple physical networks

  - Universal addressing scheme

  - Universal service

- All details of physical networks hidden from users and application programs

A protocol suite for internetworking

- The TCP/IP Internet Protocols or, simply, TCP/IP is the mostly widely used internetworking protocol suite

- First internetworking protocol suite

- Internet concept (originally called catenet developed in conjunction with TCP/IP

- Initially funded through ARPA

- Picked up by NSF

- Described in Where Wizards Stay Up Late

Internetworking protocols

- Others include IPX, VINES, AppleTalk

- TCP/IP is by far the most widely used

- Vendor and platform independent

- Used in the Internet - 20 million computers in 82 countries

TCP/IP layering

- OSI 7-layer model does not include internetworking

- TCP/IP layering model includes five layers

Layer 5: Application

Corresponds to ISO model layers 6 and 7; used for communication among applications

Layer 4: Transport

Corresponds to layer 4 in the ISO model; provides reliable delivery of data

Layer 3: Internet

Defines uniform format of packets forwarded across networks of different technologies and rules for forwarding packets in routers

Layer 2: Network

Corresponds to layer 2 in the ISO model; defines formats for carrying packets in hardware frames

Layer 1: Hardware

Corresponds to layer 1 in the ISO model; defines basic networking hardware

Hosts, routers and protocol layers

- A host computer or host is any system attached to an internet that runs applications

- Hosts may be supercomputers or toasters

- TCP/IP allows any pair of hosts on an internet communicate directly

- Both hosts and routers have TCP/IP stacks

  - Hosts typically have one interface and don't forward packets

  - Routers don't need layers 4 and 5 for packet forwarding

Summary

- An internet is a collection of physical networks interconnected into a single virtual network

- Routers provide the physical interconnection and forward packets between networks

- Hosts communicate across multiple network through packets forwarded by routers

- TCP/IP is the most widely used internetworking protocol suite

Chapter 16 - IP Protocol Addresses

| Section | Title |
| --- | --- |
| 1 | Motivation |
| 2 | TCP/IP addresses |
| 3 | IP address hierarchy |
| 4 | Network and host numbers |
| 5 | Porperties of IP addresses |
| 6 | Designing the format of IP addresses |
| 7 | Classes of addresses |
| 8 | Using IP address classes |
| 9 | Dotted decimal notation |
| 10 | Bucknell's IP addresses |
| 11 | Address classes at a glance |
| 12 | Networks and hosts in each class |
| 13 | Internet address allocation |
| 14 | Example |
| 15 | Special IP addresses |
| 16 | Berkeley broadcast address |
| 17 | Routers and IP addressing |
| 18 | Multi-homed hosts |
| 19 | Summary |

Motivation

- One key aspect of virtual network is single, uniform address format

- Can't use hardware addresses because different technologies have different address formats

- Address format must be independent of any particular hardware address format

- Sending host puts destination internet address in packet

- Destination address can be interpreted by any intermediate router

- Routers examine address and forward packet on to the destination

TCP/IP addresses

- Addressing in TCP/IP is specified by the Internet Protocol (IP)

- Each host is assigned a 32-bit number

- Called the IP address or Internet address

- Unique across entire Internet

IP address hierarchy

- Each IP address is divided into a prefix and a suffix

  - Prefix identifies network to which computer is attached

  - Suffix identifies computer within that network

- Address format makes routing efficient

Network and host numbers

- Every network in a TCP/IP internet is assigned a unique network number

- Each host on a specific network is assigned a host number or host address that is unique within that network

- Host's IP address is the combination of the network number (prefix) and host address (suffix)

Porperties of IP addresses

- Network numbers are unique

- Host addresses may be reused on different networks; combination of network number prefix and host address suffix will be unique

- Assignment of network numbers must be coordinated globally; assignment of host addresses can be managed locally

Designing the format of IP addresses

- IP designers chose 32-bit addresses

- Allocate some bits for prefix, some for suffix

  - Large prefix, small suffix - many networks, few hosts per network

  - Small prefix, large suffix - few networks, many hosts per network

- Because of variety of technologies, need to allow for both large and small networks

Classes of addresses

- Designers chose a compromise - multiple address formats that allow both large and small prefixes

- Each format is called an address class

- Class of an address is identified by first four bits

Using IP address classes

- Class A, B and C are primary classes

- Used for ordinary host addressing

- Class D is used for multicast, a limited form of broadcast

  - Internet hosts join a multicast group

  - Packets are delivered to all members of group

  - Routers manage delivery of single packet from source to all members of multicast group

  - Used for mbone (multicast backbone)

- Class E is reserved

Dotted decimal notation

- Class A, B and C all break between prefix and suffix on byte boundary

- Dotted decimal notation is a convention for representing 32-bit internet addresses in decimal

- Convert each byte of address into decimal; display separated by periods (``dots'')

Bucknell's IP addresses

- Bucknell has a single Class B network: 134.82.0.0

- All hosts at Bucknell have 134.82 prefix:

  - 134.82.7.4 - coral

  - 134.82.56.108 - leo

  - 134.82.131.3 - charcoal

- Suffix bytes are used to determine local network and host through sub-netting

Address classes at a glance

- While dotted decimal makes separating network address from host address easier, determining class is not so obvious

- Look at first dotted decimal number, and use this table:

Networks and hosts in each class

- Classing scheme does not yield equal number of networks in each class

- Class A:

  - First bit must be 0

  - 7 remaining bits identify Class A net

  - 27 (= 128) possible class A nets

Internet address allocation

- Addresses in the Internet are not used efficiently

- Bucknell is typical, using 2,000-3,000 out of possible 2^16

- Large organizations may not be able to get as many addresses in the Internet as they need

- Example - UPS needs addresses for millions of computers

- Solution - set up private internet and allocate addresses from entire 32-bit address space

Example

- Select address class for each network depending on expected number of hosts

- Assign network numbers from appropriate classes

- Assign host suffixes to form internet addresses for all hosts

Special IP addresses

Berkeley broadcast address

- First BSD implementation (Berkeley Software Distribution) of UNIX used all 0s for broadcast instead of all 1s

- This non-standard implementation spread with BSD UNIX

- Still in common use today

``There are two major developments that have come out of Berkeley: BSD UNIX and LSD. This is not a coincidence.'' - Anon.

Routers and IP addressing

- IP address depends on network address

- What about routers - connected to two networks?

- IP address specifies an interface, or network attachment point, not a computer

- Router has multiple IP addresses - one for each interface

Multi-homed hosts

- Hosts (that do not forward packets) can also be connected to multiple networks

- Can increase reliability and performance

- Multi-homed hosts also have one address for each interface

Summary

- Virtual network needs uniform addressing scheme, independent of hardware

- IP address is a 32-bit address; each interface gets a unique IP address

- IP address is composed of a network address and a host address

- Network addresses are divided into three primary classes: A, B and C

- Dotted decimal notation is a standard format for Internet addresses: 134.82.11.70

- Routers have multiple addresses - one for each interface

Chapter 17 - Binding Protocol Addresses

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Address translation |
| 3 | Address resolution |
| 4 | Address resolution (continued) |
| 5 | Address resolution techniques |
| 6 | Table lookup |
| 7 | Table lookup (continued) |
| 8 | Closed-form computation |
| 9 | Dynamic resolution |
| 10 | Dynamic resolution techniques |
| 11 | ARP |
| 12 | ARP message exchange |
| 13 | ARP example |
| 14 | ARP message contents |
| 15 | ARP message format |
| 16 | Sending an ARP message |
| 17 | Caching ARP responses |
| 18 | Identifying ARP frames |
| 19 | Processing ARP messages |
| 20 | Layering and address resolution |
| 21 | Summary |

Introduction

- Upper levels of protocol stack use protocol addresses

- Network hardware must use hardware address for eventual delivery

- Protocol address must be translated into hardware address for delivery; will discuss three methods

Address translation

- Upper levels use only protocol addresses

  - "Virtual network" addressing scheme

  - Hides hardware details

- Translation occurs at data link layer

  - Upper layer hands down protocol address of destination

  - Data link layer translates into hardware address for use by hardware layer

Address resolution

- Finding hardware address for protocol address:

  - address resolution

  - Data link layer resolves protocol address to hardware address

- Resolution is local to a network

- Network component only resolves address for other components on same network

Address resolution (continued)

- A resolves protocol address for B for protocol messages from an application on A sent to an application on B

- A does not resolve a protocol address for F

  - Through the internet layer, A delivers to F by routing through R1 and R2

  - A resolves R1 hardware address

- Network layer on A passes packet containing destination protocol address F for delivery to R1

Address resolution techniques

- Association between a protocol address and a hardware address is called a binding

- Three techniques:

  - Table lookup

    - Bindings stored in memory with protocol address as key

    - Data link layer looks up protocol address to find hardware address

  - Closed-form computation

    - Protocol address based on hardware address

    - Data link layer derives hardware address from protocol address

  - Dynamic

    - Network messages used for "just-in-time" resolution

    - Data link layer sends message requesting hardware address; destination responds with its hardware address

Table lookup

- Use a simple list containing IP address and hardware address for each host on net

- Search on IP address and extract corresponding hardware address

- Note that all IP addresses have same prefix; can save space by dropping prefix

Table lookup (continued)

- Sequential search may be prohibitively expensive (O(n2))

- Can use indexing or hashing for O(n) lookup

  - Indexing - use hostid part of IP address as list (array) index

  - Hashing - use hashing function on hostid to generate list index

Closed-form computation

- If hardware technology uses small, configurable hardware address, network administrator can choose hardware address based on IP address

- Example - hardware uses one octet address that can be configured

- Simply choose hardware address to be hostid

- Now, any host can determine hardware address as:

hardware_address = ip_address & 0xff

Dynamic resolution

- Use network to resolve IP addresses

- Message exchange with other computer(s) returns hardware address to source

- Two designs:

  - Server-based - computer sends message to server to resolve address

    - List of servers

    - Broadcast to locate servers

  - Distributed - all computers participate; destination provides hardware address to host

Dynamic resolution techniques

- Server-based - centralized, easier to manage, used on non-broadcast media (e.g., ATM)

- Distributed - requires no dedicated computers, no administration

ARP

- IP uses distributed resolution technique

- Address Resolution Protocol (ARP) - part of TCP/IP protocol suite

- Two-part protocol

  - Request from source asking for hardware address

  - Reply from destination carrying hardware address

ARP message exchange

- ARP request message dropped into hardware frame and broadcast

- Uses separate protocol type in hardware frame (ethernet = 806)

- Sender inserts IP address into message and broadcast

- Every other computer examines request

- Computer whose IP address is in request responds

  - Puts hardware address in response

  - Unicasts to sender

- Original requester can then extract hardware address and send IP packet to destination

ARP example

ARP message contents

- Maps protocol address to hardware address

- Both protocol address and hardware address sizes are variable

  - Ethernet = 6 octets

  - IP = 4 octets

- Can be used for other protocols and hardware types

ARP message format

- HARDWARE ADDRESS TYPE = 1 for Ethernet

- PROTOCOL ADDRESS TYPE = 0x0800 for IP

- OPERATION = 1 for request, 2 for response

- Contains both target and sender mappings from protocol address to hardware address

  - Request sets hardware address of target to 0

  - Target can extract hardware address of sender (saving an ARP request)

  - Target exchanges sender/target in response

Sending an ARP message

- Sender constructs ARP message

- ARP message carried as data in hardware frame - encapsulation

Caching ARP responses

- Using ARP for each IP packet adds two packets of overhead for each IP packet

- Computer caches ARP responses

  - Flushes cache at system startup

  - Entries discarded periodically

- Cache searched prior to sending ARP request

Identifying ARP frames

- Uses separate frame type

- Ethernet uses type 0x0806

Processing ARP messages

- Receiver extracts sender's hardware address and updates local ARP table

- Receiver checks operation - request of response

- Response:

  - Adds sender's address to local cache

  - Sends pending IP packet(s)

- Request:

  - If receiver is target, forms response

  - Unicasts to sender

  - Adds sender's address to local cache

- Note:

  - Target likely to respond "soon"

  - Computers have finite storage for ARP cache

  - Only target adds sender to cache; others only update if target already in cache

Layering and address resolution

- Address resolution (ARP) is a network interface layer function

- Protocol addresses used in all higher layers

- Hides ugly details and allows generality in upper layers

Summary

- Address resolution - translates protocol address to hardware address

  - Static - table lookup

  - Computation - extract hardware address from protocol address

  - Dynamic - use network messages to resolve protocol address

- ARP - TCP/IP protocol for address resolution

Chapter 18 - IP Datagrams and Datagram Forwarding

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Connectionless service |
| 3 | Virtual packets |
| 4 | IP datagram format |
| 5 | Forwarding datagrams |
| 6 | Routing example |
| 7 | Routing table |
| 8 | Default routes |
| 9 | Routing tables and address masks |
| 10 | Address masks |
| 11 | Forwarding, destination address and next-hop |
| 12 | Best-effort delivery |
| 13 | IP datagram header format |
| 14 | IP datagram header fields |
| 15 | IP datagram options |
| 16 | Summary |

Introduction

- Fundamental Internet communication service

- Format of packets

- Processing of packets by routers

  - Forwarding

  - Delivery

Connectionless service

- End-to-end delivery service is connectionless

- Extension of LAN abstraction

  - Universal addressing

  - Data delivered in packets (frames), each with a header

- Combines collection of physical networks into single, virtual network

- Transport protocols use this connectionless service to provide connectionless data delivery (UDP) and connection-oriented data delivery (TCP)

Virtual packets

- Packets server same purpose in internet as frames on LAN

- Each has a header

- Routers (formerly gateways) forward between physical networks

- Packets have a uniform, hardware-independent format

  - Includes header and data

  - Can't use format from any particular hardware

- Encapsulated in hardware frames for delivery across each physical network

IP datagram format

- Formally, the unit of IP data delivery is called a datagram

- Includes header area and data area

- Datagrams can have different sizes

  - Header area usually fixed (20 octets) but can have options

  - Data area can contain between 1 octet and 65,535 octets (216 - 1)

  - Usually, data area much larger than header

Forwarding datagrams

- Header contains all information needed to deliver datagram to destination computer

  - Destination address

  - Source address

  - Identifier

  - Other delivery information

- Router examines header of each datagram and forwards datagram along path to destination

Routing example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap18/chap18_6.html

Shockwave

Routing table

- For efficiency, information about forwarding is stored in a routing table

  - Initialized at system initialization

  - Must be updated as network topology changes

- Contains list of destination networks and next hop for each destination

Default routes

- Routing table kept small by listing destination networks rather than hosts

- Can be further reduced through default route

  - Entry used if destination network not explicitly listed in routing table

  - E.g., Bucknell uses default routes for all off-campus networks

Routing tables and address masks

- In practice, additional information is kept in routing table

- Destination stored as network address

- Next hop stored as IP address of router

- Address mask defines how many bits of address are in prefix

  - Prefix defines how much of address used to identify network

  - E.g., class A mask is 255.0.0.0

  - Used for subnetting

Address masks

- To identify destination network, apply address mask to destination address and compare to network address in routing table

- Can use Boolean and

if ((Mask[i] & D) == Dest[i]) forward to NextHop[i]

- Consider 128.1.15.26:

Forwarding, destination address and next-hop

- Destination address in IP datagram is always ultimate destination

- Router looks up next-hop address and forwards datagram

- Network interface layer takes two parameters:

  - IP datagram

  - Next-hop address

- Next-hop address never appears in IP datagram

Best-effort delivery

- IP provides service equivalent to LAN

- Does not guarantee to prevent

  - Duplicate datagrams

  - Delayed or out-of-order delivery

  - Corruption of data

  - Datagram loss

- Reliable delivery provided by transport layer

- Network layer - IP - can detect and report errors without actually fixing them

  - Network layer focuses on datagram delivery

  - Application layer not interested in differentiating among delivery problems at intermediate routers

IP datagram header format

IP datagram header fields

- VERS - version of IP (currently 4)

- H. LEN - header length (in units of 32 bits)

- SERVICE TYPE - sender's preference for low latency, high reliability (rarely used)

- TOTAL LENGTH - total octets in datagram

- IDENT, FLAGS, FRAGMENT OFFSET - used with fragmentation

- TTL - time to live; decremented in each router; datagram discarded when TTL = 0

- TYPE - type of protocol carried in datagram; e.g., TCP, UDP

- HEADER CHECKSUM - 1s complement of 1s complement sum

- SOURCE, DEST IP ADDRESS - IP addresses of original source and ultimate destination

IP datagram options

- Several options can be added to IP header:

  - Record route

  - Source route

  - Timestamp

- Header with no options has H. LEN field value 5; data begins immediately after DESTINATION IP ADDRESS

- Options added between DESTINATION IP ADDRESS and data in multiples of 32 bits

- Header with 96 bits of options has H. LEN field value 8

Summary

- Basic unit of delivery in TCP/IP is IP datagram

- Routers use destination address in IP datagram header to determine next-hop

- Forwarding information stored in routing table

- IP datagram header has 40 octets of fixed field information and (possibly) options

Chapter 19 - IP Encapsulation, Fragmentation and Reassembly

| Section | Title |
| --- | --- |
| 1 | Datagram transmission and frames |
| 2 | Encapsulation |
| 3 | Encapsulation across multiple hops |
| 4 | Internet encapsulation (example) |
| 5 | MTU |
| 6 | MTU and datagram transmission |
| 7 | MTU and heterogeneous networks |
| 8 | Fragmentation |
| 9 | Fragmentation (details) |
| 10 | Datagram reassembly |
| 11 | Fragment identification |
| 12 | Fragment loss |
| 13 | Fragmenting a fragment |
| 14 | Summary |

Datagram transmission and frames

- IP internet layer

  - Constructs datagram

  - Determines next hop

  - Hands to network interface layer

- Network interface layer

  - Binds next hop address to hardware address

  - Prepares datagram for transmission

- But ... hardware frame doesn't understand IP; how is datagram transmitted?

Encapsulation

- Network interface layer encapsulates IP datagram as data area in hardware frame

  - Hardware ignores IP datagram format

  - Standards for encapsulation describe details

- Standard defines data type for IP datagram, as well as others (e.g., ARP)

- Receiving protocol stack interprets data area based on frame type

Encapsulation across multiple hops

- Each router in the path from the source to the destination:

  - Unencapsulates incoming datagram from frame

  - Processes datagram - determines next hop

  - Encapsulates datagram in outgoing frame

- Datagram may be encapsulated in different hardware format at each hop

- Datagram itself is (almost!) unchanged

Internet encapsulation (example)

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap19/chap19_4.html

Shockwave

MTU

- Every hardware technology specification includes the definition of the maximum size of the frame data area

- Called the maximum transmission unit (MTU)

- Any datagram encapsulated in a hardware frame must be smaller than the MTU for that hardware

MTU and datagram transmission

- IP datagrams can be larger than most hardware MTUs

  - IP: 216 - 1

  - Ethernet: 1500

  - Token ring: 2048 or 4096

- Source can simply limit IP datagram size to be smaller than local MTU

  - Must pass local MTU up to TCP for TCP segments

  - What about UDP?

MTU and heterogeneous networks

- An internet may have networks with different MTUs

- Suppose downstream network has smaller MTU than local network?

Fragmentation

- One technique - limit datagram size to smallest MTU of any network

- IP uses fragmentation - datagrams can be split into pieces to fit in network with small MTU

- Router detects datagram larger than network MTU

  - Splits into pieces

  - Each piece smaller than outbound network MTU

Fragmentation (details)

- Each fragment is an independent datagram

  - Includes all header fields

  - Bit in header indicates datagram is a fragment

  - Other fields have information for reconstructing original datagram

  - FRAGMENT OFFSET gives original location of fragment

- Router uses local MTU to compute size of each fragment

- Puts part of data from original datagram in each fragment

- Puts other information into header

Datagram reassembly

- Reconstruction of original datagram is call reassembly

- Ultimate destination performs reassembly

- Fragments may arrive out of order; header bit identifies fragment containing end of data from original datagram

- Fragment 3 identified as last fragment

Fragment identification

- How are fragments associated with original datagram?

- IDENT field in each fragment matches IDENT field in original datagram

- Fragments from different datagrams can arrive out of order and still be sorted out

Fragment loss

- IP may drop fragment

- What happens to original datagram?

  - Destination drops entire original datagram

- How does destination identify lost fragment?

  - Sets timer with each fragment

  - If timer expires before all fragments arrive, fragment assumed lost

  - Datagram dropped

- Source (application layer protocol) assumed to retransmit

Fragmenting a fragment

- Fragment may encounter subsequent network with even smaller MTU

- Router fragments the fragment to fit

- Resulting (sub)fragments look just like original fragments (except for size)

- No need to reassemble hierarchically; (sub)fragments include position in original datagram

Summary

- IP uses encapsulation to transmit datagrams in hardware frames

- Network technologies have an MTU

- IP uses fragmentation to carry datagrams larger than network MTU

Chapter 20 - IPv6

| Section | Title |
| --- | --- |
| 1 | Introduction - the future of IP |
| 2 | Success of IP |
| 3 | Motivation for change |
| 4 | Name and versions number |
| 5 | New features |
| 6 | IPv6 datagram format |
| 7 | IPv6 base header format |
| 8 | IPv6 NEXT HEADER |
| 9 | Parsing IPv6 headers |
| 10 | Fragmentation |
| 11 | Fragmentation and path MTU |
| 12 | Use of multiple headers |
| 13 | IPv6 addressing |
| 14 | IPv6 address notation |
| 15 | Summary |

Introduction - the future of IP

- Current version of IP - version 4 - is 20 years old

- IPv4 has shown remarkable ability to move to new technologies

- IETF has proposed entirely new version to address some specific problems

Success of IP

- IP has accommodated dramatic changes since original design

  - Basic principles still appropriate today

  - Many new types of hardware

  - Scale

- Scaling

  - Size - from a few tens to a few tens of millions of computers

  - Speed - from 56Kbps to 1Gbps

  - Increased frame size in hardware

Motivation for change

- Address space

  - 32 bit address space allows for over a million networks

  - But...most are Class C and too small for many organizations

  - 214 Class B network addresses already almost exhausted (and exhaustion was first predicted to occur a couple of years ago)

- Type of service

  - Different applications have different requirements for delivery reliability and speed

  - Current IP has type of service that's not often implemented

- Multicast

Name and versions number

- Preliminary versions called IP - Next Generation (IPng)

- Several proposals all called IPng

- One was selected and uses next available version number (6)

- Result is IP version 6 (IPv6)

New features

- Address size - IPv6 addresses are 128bits

- Header format - entirely different

- Extension headers - Additional information stored in optional extension headers, followed by data

- Support for audio and video - flow labels and quality of service allow audio and video applications to establish appropriate connections

- Extensible - new features can be added more easily

IPv6 datagram format

IPv6 base header format

- Contains less information than IPv4 header

- NEXT HEADER points to first extension header

- FLOW LABEL used to associate datagrams belonging to a flow or communication between two applications

  - Traffic class

  - Specific path

  - Routers use FLOW LABEL to forward datagrams along prearranged path

IPv6 NEXT HEADER

Parsing IPv6 headers

- Base header is fixed size - 40 octets

  - NEXT HEADER field in base header defines type of header

  - Appears at end of fixed-size base header

- Some extensions headers are variable sized

  - NEXT HEADER field in extension header defines type

  - HEADER LEN field gives size of extension header

Fragmentation

- Fragmentation information kept in separate extension header

- Each fragment has base header and (inserted) fragmentation header

- Entire datagram, including original header may be fragmented

Fragmentation and path MTU

- IPv6 source (not intermediate routers) responsible for fragmentation

  - Routers simply drop datagrams larger than network MTU

  - Source must fragment datagram to reach destination

- Source determines path MTU

  - Smallest MTU on any network between source and destination

  - Fragments datagram to fit within that MTU

- Uses path MTU discovery

  - Source sends probe message of various sizes until destination reached

  - Must be dynamic - path may change durin

Use of multiple headers

- Efficiency - header only as large as necessary

- Flexibility - can add new headers for new features

- Incremental development - can add processing for new features to testbed; other routers will skip those headers

IPv6 addressing

- 128-bit addresses

- Includes network prefix and host suffix

- No address classes - prefix/suffix boundary can fall anywhere

- Special types of addresses:

  - unicast - single destination computer

  - multicast - multiple destinations; possibly not at same site

  - cluster - collection of computers with same prefix; datagram is delivered to one out of cluster

- IPv4 broadcast flavors are subsets of multicast

- Cluster addressing allows for duplication of services

IPv6 address notation

- 128-bit addresses unwieldy in dotted decimal; requires 16 numbers

105.220.136.100.255.255.255.255.0.0.18.128.140.10.255.255

- Groups of 16-bit numbers in hex separated by colons - colon hexadecimal (or colon hex)

69DC:8864:FFFF:FFFF:0:1280:8C0A:FFFF

- Zero-compression - series of zeroes indicated by two colons

FF0C:0:0:0:0:0:0:B1

FF0C::B1

- IPv6 address with 96 leading zeros is interpreted to hold an IPv4 address

Summary

- IPv4 basic abstractions have been very successful

- IPv6 carries forward many of those abstraction... but, all the details are changed

  - 128-bit addresses

  - Base and extension headers

  - Source does fragmentation

  - New types of addresses

  - Address notation

Chapter 21 - ICMP

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Error detection |
| 3 | Error reporting |
| 4 | Types of messages |
| 5 | ICMP message transport |
| 6 | ICMP and reachability |
| 7 | ICMP and internet routes |
| 8 | ICMP and path MTU discovery |
| 9 | ICMP and router discovery |
| 10 | ICMP redirect |
| 11 | Summary |

Introduction

- IP provides best-effort delivery

- Delivery problems can be ignored; datagrams can be "dropped on the floor"

- Internet Control Message Protocol (ICMP) provides error-reporting mechanism

Error detection

- Internet layer can detect a variety of errors:

  - Checksum (header only!)

  - TTL expires

  - No route to destination network

  - Can't deliver to destination host (e.g., no ARP reply)

- Internet layer discards datagrams with problems

- Some - e.g., checksum error - can't trigger error messages

Error reporting

- Some errors can be reported

  - Router sends message back to source in datagram

  - Message contains information about problem

- Encapsulated in IP datagram

Types of messages

- Internet Control Message Protocol (ICMP) defines error and informational messages

- Error messages:

  - Source quench

  - Time exceeded

  - Destination unreachable

  - Redirect

  - Fragmentation required

- Informational messages:

  - Echo request/reply

  - Address mask request/reply

  - Router discovery

ICMP message transport

- ICMP encapsulated in IP

- But ... how can that work?

- ICMP messages sent in response to incoming datagrams with problems

- ICMP message not sent for ICMP message

ICMP and reachability

- An internet host, A, is reachable from another host, B, if datagrams can be delivered from A to B

- ping program tests reachability - sends datagram from B to A that A echoes back to B

- Uses ICMP echo request and echo reply messages

- Internet layer includes code to reply to incoming ICMP echo request messages

ICMP and internet routes

- List of all routers on path from A to B is called the route from A to B

- traceroute uses UDP to non-existent port and TTL field to find route via expanding ring search

- Sends ICMP echo messages with increasing TTL

  - Router that decrements TTL to 0 sends ICMP time exceeded message, with router's address as source address

  - First, with TTL 1, gets to first router, which discards and sends time exceeded message

  - Next, with TTL 1, gets through first router to second router

  - Continue until message from destination received

- traceroute must accommodate varying network delays

- Must also accommodate dynamically changing routes

ICMP and path MTU discovery

- Fragmentation should be avoided

- How can source configure outgoing datagrams to avoid fragmentation?

- Source determines path MTU - smallest network MTU on path from source to destination

- Source probes path using IP datagrams with don't fragment flag

- Router responds with ICMP fragmentation required message

- Source sends smaller probes until destination reached

ICMP and router discovery

- Router can fail, causing "black-hole" or isolating host from internet

- ICMP router discovery used to find new router

- Host can broadcast request for router announcements to auto-configure default route

- Host can broadcast request if router fails

- Router can broadcast advertisement of existence when first connected

ICMP redirect

- Default route may cause extra hop

- Router that forwards datagram on same interface sends ICMP redirect

- Host installs new route with correct router as next hop

Summary

- Internet layer provides best-effort delivery service

- May choose to report errors for some problems

- ICMP provides error message service

Chapter 22 - Transport Protocols

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | User Datagram Protocol |
| 3 | UDP and TCP/IP layering |
| 4 | UDP headers |
| 5 | Selecting UDP port numbers |
| 6 | Well-known port numbers |
| 7 | TCP |
| 8 | Features of TCP |
| 9 | Using IP for data delivery |
| 10 | Delvering TCP |
| 11 | TCP and reliable delivery |
| 12 | Lost packets |
| 13 | TCP segments and sequence numbers |
| 14 | Acknowledgments |
| 15 | Setting the timeout |
| 16 | RTOs for different network delays |
| 17 | Picking a timeout value |
| 18 | Computing RTT and RTO |
| 19 | Measuring RTT |
| 20 | Karn's algorithm |
| 21 | TCP sliding window |
| 22 | Sliding window with acknowledgments |
| 23 | Sliding window example |
| 24 | Sliding window with lost segment |
| 25 | Flow control with sliding window |
| 26 | Silly window syndrome |
| 27 | TCP segment format |
| 28 | Three-way handshake |
| 29 | Closing a connection |
| 30 | Opening a connection |
| 31 | Closing a connection |
| 32 | Congestion control |
| 33 | Summary |

Introduction

- Internet Protocol (IP) provides ``unreliable datagram service'' between hosts

- Transport protocols provide end-to-end delivery between endpoints of a connection; e.g., processes or programs

- User Datagram Protocol (UDP) provides datagram service

- Transmission Control Protocol (TCP) provides reliable data delivery

User Datagram Protocol

- UDP delivers independent messages, called datagrams between applications or processes on host computers

  - ``Best effort'' delivery - datagrams may be lost, delivered out of order, etc.

  - Checksum (optionally) guarantees integrity of data

- For generality, endpoints of UDP are called protocol ports or ports

- Each UDP data transmission identifies the internet address and port number of the destination and the source of the message

- Destination port and source port may be different

UDP and TCP/IP layering

- Transport protocols use IP to provide data delivery for application protocols

Application

Transport UDP, TCP

Internet

Network interface

Hardware

UDP headers

- UDP datagrams have a header that follows the hardware and IP headers:

| Hardware | IP | UDP | Application | Data |
| --- | --- | --- | --- | --- |

- UDP header is very simple:

  - Port numbers

  - Message length

  - Checksum

| UDP source port | UDP destination port |
| --- | --- |
| UDP message length | UDP checksum |
| Data |  |

Selecting UDP port numbers

- Communicating computers must agree on a port number

  - ``Server'' opens selected port and waits for incoming messages

  - ``Client'' selects local port and sends message to selected port

- Services provided by many computers use reserved, well-known port numbers:

  - ECHO

  - DISCARD

  - NTP

- Other services use dynamically assigned port numbers

Well-known port numbers

| Port | Name | Description |
| --- | --- | --- |
| 7 | echo | Echo input back to sender |
| 9 | discard | Discard input |
| 11 | systat | System statistics |
| 13 | daytime | Time of day (ASCII) |
| 17 | quote | Quote of the day |
| 19 | chargen | Character generator |
| 37 | time | System time (seconds since 1970) |
| 53 | domain | DNS |
| 69 | tftp | Trivial File Transfer Protocol (TFTP) |
| 123 | ntp | Network Time Protocol (NTP) |
| 161 | snmp | Simple Network Management Protocol (SNMP) |

TCP

- Transmission Control Protocol (TCP) is most widely used transport protocol

- Provides reliable data delivery by using IP unreliable datagram delivery

- Compensates for loss, delay, duplication and similar problems in Internet components

- Reliable delivery is high-level, familiar model for construction of applications

Features of TCP

- Connection oriented: Application requests connection to destination and then uses connection to deliver data to transfer data

- Point-to-point: A TCP connection has two endpoints

- Reliability: TCP guarantees data will be delivered without loss, duplication or transmission errors

- Full duplex: The endpoints of a TCP connection can exchange data in both directions simultaneously

- Stream interface: Application delivers data to TCP as a continuous stream, with no record boundaries; TCP makes no guarantees that data will be received in same blocks as transmitted

- Reliable connection startup: Three-way handshake guarantees reliable, synchronized startup between endpoints

- Graceful connection shutdown: TCP guarantees delivery of all data after endpoint shutdown by application

Using IP for data delivery

- TCP uses IP for data delivery (like UDP)

- Endpoints are identified by ports (like UDP)

  - Allows multiple connections on each host

  - Ports may be associated with an application or a process

- IP treats TCP like data and does not interpret any contents of the TCP message

Delvering TCP

- TCP travels in IP datagrams

- Internet routers only look at IP header to forward datagrams

- TCP at destination interprets TCP messages

TCP and reliable delivery

- TCP uses many techniques described earlier to provide reliable delivery

- Recovers from

  - Lost packets

  - Duplicate packets

  - Delayed packets

  - Corrupted data

  - Transmission speed mismatches

  - Congestion

  - System reboots

Lost packets

- TCP uses positive acknowledgment with retransmission to achieve reliable data delivery

- Recipient sends acknowledgment control messages (ACK) to sender to verify successful receipt of data

- Sender sets timer when data transmitted; if timer expires before acknowledgment arrives, sender retransmits (with new timer)

TCP segments and sequence numbers

- Application delivers arbitrarily large chunks of data to TCP as a``stream''

- TCP breaks this data into segments, each of which fits into an IP datagram

- Original stream is numbered by bytes

- Segment contains sequence number of data bytes

Acknowledgments

- Receiver sends segment with sequence number of acknowledged data (not segments)

- One ACK can acknowledge many segments

Setting the timeout

- Inappropriate timeout can cause poor performance:

  - Too long - sender waits longer than necessary before retransmitting

  - Too short - sender generates unnecessary traffic

- Timeout must be different for each connection and set dynamically

  - Host on same LAN should have shorter timeout than host 20 hops away

  - Delivery time across internet may change over time; timeout must accommodate changes

RTOs for different network delays

Picking a timeout value

- Timeout should be based on round trip time (RTT)

- Sender can't know RTT of any packet before transmission

- Sender picks retransmission timeout (RTO) based on previous RTTs

- Specific method is call adaptive retransmission algorithm

Computing RTT and RTO

- Weighted average for RTT:

RTTnew = (alpha*RTTold) + ((1 - alpha)*RTTsample))

- Computation of RTO:

RTO = beta*RTTnew

Measuring RTT

- RTT measured by observing difference between time of transmission and arrival of acknowledgment

- However - acknowledgments carry no information about which packet is acknowledged

- Sender cannot determine whether acknowledgment is from original transmission or retransmission

  - Choosing original transmission overestimates RTT

  - Choosing retransmission underestimates RTT

Karn's algorithm

- How to choose between original and retransmission? Answer: choose neither!

- Karn's algorithm specifies that sender ignores RTTs for retransmitted segments

- How will RTT get updated if internet round trip time increases?

- Karn's algorithm specifies that RTO is separated from RTT when retransmission occurs

- RTO doubles for each new message until ACK arrives with no retransmission

TCP sliding window

- TCP uses sliding window for flow control

- Receiver specifies window

  - Called window advertisement

  - Specifies which bytes in the data stream can be sent

  - Carried in segment along with ACK

- Sender can transmit any bytes, in any size segment, between last acknowledged byte and within window size

Sliding window with acknowledgments

Sliding window example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap22/chap22_23.html

Shockwave

Sliding window with lost segment

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap22/chap22_24.html

Shockwave

Flow control with sliding window

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap22/chap22_25.html

Shockwave

Silly window syndrome

- Under some circumstances, sliding window can result in transmission of many small segments

- If receiver window full, and receiving application consumes a few data bytes, receiver will advertise small window

- Sender will immediately send small segment to fill window

- Inefficient in processing time and network bandwidth

- Solutions:

  - Receiver delays advertising new window

  - Sender delays sending data when window is small

TCP segment format

- TCP segment has header containing:

- Same header format used in both directions

- Segment can carry both data and acknowledgment

Three-way handshake

- TCP uses three-way handshake for reliable connection establishment and termination

  - Host 1 sends segment with SYN bit set and random sequence number

  - Host 2 responds with segment with SYN bit set, acknowledgment to Host 1 and random sequence number

  - Host 1 responds with acknowledgment

- TCP will retransmit lost segments

- Random sequence numbers ensure synchronization between endpoints

Closing a connection

Opening a connection

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap22/chap22_30.html

Shockwave

Closing a connection

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap22/chap22_31.html

Shockwave

Congestion control

- Excessive traffic can cause packet loss

  - Transport protocols respond with retransmission

  - Excessive retransmission can cause congestion collapse

- TCP interprets packet loss as an indicator of congestion

- Sender uses TCP congestion control and slows transmission of packets

  - Sends single packet

  - If acknowledgment returns without loss, sends two packets

  - When TCP sends one-half window size, rate of increase slows

Summary

- UDP provides end-to-end best-effort message delivery

  - IP used for delivery to destination host

  - Protocol ports demultiplex to destination application

- TCP provides end-to-end reliable bytestream delivery

  - IP used for delivery to destination host

  - Protocol ports demultiplex to destination application

  - Additional techniques develop reliable delivery from IP messages

- Positive acknowledgment with retransmission

- Sequence numbers detect missing, duplicate and out-of-order data

- Sliding window flow control

- Three-way handshake

- Congestion control

Chapter 23 - Client-Server Interaction

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Internet protocols and network applications |
| 3 | Establising contact through internet protocols |
| 4 | Client-server paradigm |
| 5 | Characteristics of client |
| 6 | Characteristics of server |
| 7 | ``Server-class'' computers |
| 8 | Message exchanges |
| 9 | Transport protocols and client-server paradigm |
| 10 | Multiple services on one computer |
| 11 | Identifying a service |
| 12 | Multiple servers for one service |
| 13 | Master-slave servers |
| 14 | Selecting from multiple servers |
| 15 | Connection-oriented and connectionless transport |
| 16 | Client-server interactions |
| 17 | Summary |

Introduction

- Application-level protocols provide high-level services

  - DNS

  - Electronic mail

  - Remote login

  - FTP

  - World Wide Web

- All of these applications use client-server architecture

Internet protocols and network applications

- Internet protocols provide

  - General-purpose facility for reliable data transfer

  - Mechanism for contacting hosts

- Application programs

  - Use internet protocols to contact other applications

  - Provide user-level services

Establising contact through internet protocols

- Application must interact with protocol software before contact is made

- Listening application informs local protocol software that it is ready to accept incoming messages

- Connecting application uses internet protocol to contact listener

- Applications exchange messages through resulting connection

Client-server paradigm

- Server application is ``listener''

  - Waits for incoming message

  - Performs service

  - Returns results

- Client application establishes connection

  - Sends message to server

  - Waits for return message

Characteristics of client

- Arbitrary application program

  - Becomes client when network service is needed

  - Also performs other computations

- Invoked directly by user

- Runs locally on user's computer

- Initiates contact with server

- Can access multiple services (one at a time)

- Does not require special hardware or sophisticated operating system

Characteristics of server

- Special purpose application dedicated to providing network service

- Starts at system initialization time

- Runs on a remote computer (usually centralized, shared computer)

- Waits for service requests from clients; loops to wait for next request

- Will accept requests from arbitrary clients; provides one service to each client

- Requires powerful hardware and sophisticated operating system

``Server-class'' computers

- Shared, centralized computers that run many server applications are sometimes called ``servers''

- More precisely, the applications are the ``servers'' and the computer is a ``server-class computer''

- Servers can run on very simple computers...

Message exchanges

- Typically, client and server exchange messages:

  - Client sends request, perhaps with data

  - Server send response, perhaps with data

- Client may send multiple requests; server sends multiple responses

- Server may send multiple response - imagine video feed

Transport protocols and client-server paradigm

- Clients and servers exchange messages through transport protocols; e.g., TCP or UDP

- Both client and server must have same protocol stack and both interact with transport layer

Multiple services on one computer

- Sufficiently powerful computer - fast enough processor, multi-tasking OS - may run multiple servers

- Servers run as independent processes and can manage clients simultaneously

Identifying a service

- Each service gets a unique identifier; both client and

- Server use that identifier

  - Server registers with local protocol software under the identifier

  - Client contacts protocol software for session under that identifier

- Example - TCP uses protocol port numbers as identifiers

  - Server registers under port number for service

  - Client requests session with port number for service

Multiple servers for one service

- Responding to a client request may require significant time

- Other clients must wait while earlier requests are satisfied

- Multiple servers can handle requests concurrently, completing shorter requests without waiting for longer requests

Master-slave servers

- One way to run concurrent servers is to dynamically create server processes for each client

- Master server accepts incoming requests and starts slave server for each client

- Slave handles subsequent requests from its client

- Master server then waits for next request

Selecting from multiple servers

- How do incoming messages get delivered to the correct server?

- Each transport session has two unique identifiers

  - (IP address, port number) on server

  - (IP address, port number) on client

- No two clients on one computer can use same source port

- Thus, client endpoints are unique, and server computer protocol software can deliver messages to correct server process

Connection-oriented and connectionless transport

- Which to choose?

- TCP - connection-oriented

  - Client establishes connection to server

  - Client and server exchange multiple messages of arbitrary size

  - Client terminates connection

- UDP - connectionless

  - Client constructs message

  - Client sends message to server

  - Server responds

  - Message must fit in one UDP datagram

- Some services use both

  - DNS, chargen, motd

  - Can be provided by single server

Client-server interactions

- Clients can access multiple services sequentially

- Clients may access different servers for one service

- Servers may become clients of other servers

- Circular dependencies may arise...

Summary

- Client-server paradigm used in almost every distributed computation

  - Client requests service when needed

  - Server waits for client requests

- Servers usually run on server-class computer

- Clients and servers use transport protocols to communicate

- Often, but not always, there is an application protocol

Chapter 24 - Socket Interface

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | API |
| 3 | The Socket API |
| 4 | Sockets and socket libraries |
| 5 | Sockets and UNIX I/O |
| 6 | The socket API |
| 7 | Summary of socket system calls |
| 8 | socket |
| 9 | close |
| 10 | bind |
| 11 | Socket address formats |
| 12 | listen |
| 13 | accept |
| 14 | connect |
| 15 | send |
| 16 | sendto, sendmsg |
| 17 | recv |
| 18 | recvfrom, recvmsg |
| 19 | Other procedures |
| 20 | Sockets and processes |
| 21 | Summary |

Introduction

- The socket is one form of interface between application programs and protocol software

- Widely available - program portability

- Used by both clients and servers

- Extension to UNIX file I/O paradigm

API

- Application interactions with protocol software:

  - Passive listen or active open

  - Protocol to use

  - IP address and port number

- Interface to protocol is call Application Program Interface (API)

  - Defined by programming/operating system

  - Includes collection of procedures for application program

The Socket API

- Protocols do not typically specify API

- API defined by programming system

- Allows greatest flexibility - compatibility with different programming systems

- Socket API is a specific protocol API

  - Originated with Berkeley BSD UNIX

  - Now available on Windows 95 and Windows NT, Solaris, etc.

- Not defined as TCP/IP standard; de factostandard

Sockets and socket libraries

- BSD UNIX includes sockets as system calls

- Other vendors (mostly UNIX) have followed suit

- Some systems have different API

  - Adding sockets would require changing OS

  - Added library procedures - socket library - instead

- Adds layer of software between application and operating system

  - Enhances portability

  - May hide native API altogether

Sockets and UNIX I/O

- Developed as extension to UNIX I/O system

- Uses same file descriptor address space (small integers)

- Based on open-read-write-close paradigm

  - open - prepare a file for access

  - read/write - access contents of file

  - close - gracefully terminate use of file

- Open returns a file descriptor, which is used to identify the file to read/write/close

The socket API

- Socket programming more complex than file I/O

- Requires more parameters

  - Addresses

  - Protocol port numbers

  - Type of protocol

  - New semantics

- Two techniques

  - Add parameters to existing I/O system calls

  - Create new system calls

- Sockets use a collection of new system calls

Summary of socket system calls

- socket - create a new socket

- close - terminate use of a socket

- bind - attach a network address to a socket

- listen - wait for incoming messages

- accept - begin using incoming connection

- connect - make connection to remote host

- send - transmit data through active connection

- recv - receive data through active connection

socket

descriptor = socket(protofamily, type, protocol)

- Returns socket descriptor used in subsequent calls

- protofamily selects protocol family; e.g.:

  - PF_INET - Internet protocols

  - PF_APPLETALK - AppleTalk protocols

- type selects type of communication

  - SOCK_DGRAM - connectionless

  - SOCK_STREAM - connection-oriented

- protocol specifies protocol within protocol family

  - IPPROTO_TCP - selects TCP

  - IPPROTO_UDP - selects UDP

close

close(descriptor)

- Terminates use of socket descriptor

- descriptor contains descriptor of socket to be closed

bind

bind(socket, localaddr, address)

- Initially, socket has no addresses attached

- bind selects either local, remote or both addresses

  - server binds local port number for incoming messages

  - client binds remote address and port number to contact server

Socket address formats

- Because sockets can be used for any protocols, address format is generic:

struct sockaddr {

u_char sa_len;            /* total length of address */

u_char sa_family;         /* family of the address   */

char   sa_data[14];       /* address                 */

}

- For IP protocols, sa_data hold IP address and port number:

struct sockaddr_in {

u_char  sin_len;          /* total length of address */

u_char  sin_family;       /* family of the address   */

u_short sin_port;         /* protocol port number    */

struct  in_addr sin_addr; /* IP address              */

char    sin_zero[8]       /* unused                  */

}

- First two fields match generic sockaddr structure

- Remainder are specific to IP protocols

- INADDR_ANY interpreted to mean "any" IP address

listen

listen(socket, queuesize)

- Server uses listen to wait for incoming connections

- socket identifies socket through which connections will arrive (address)

- New connection requests may arrive while server processes previous request

- Operating system can hold requests on queue

- queuesize sets upper limit on outstanding requests

accept

accept(socket, caddress, caddresslen)

- Server uses accept to accept the next connection request

- accept call blocks until connection request arrives

- Returns new socket with server's end of new connection

- Old socket remains unchanged and continues to field incoming requests

- caddress returns struct sockaddr client address; format depends on address family of socket

- caddresslen returns length of address

connect

connect(socket, saddress, saddresslen)

- Client uses connect to establish connection to server

- Blocks until connection completed (accepted)

- socket holds descriptor of socket to use

- saddress is a struct sockaddr that identifies server

- saddresslen gives length of saddress

- Usually used with connection-oriented transport protocol

- Can be used with connectionless protocol

  - Marks local socket with server address

  - Implicitly identifies server for subsequent messages

send

send(socket, data, length, flags)

- Used to send data through a connected socket

- socket identifies socket

- data points to data to be sent

- length gives length of data (in bytes)

- flags indicate special options

sendto, sendmsg

sendto(socket, data, length, flags, destaddress, addresslen)

sendmsg(socket, msgstruct, flags)

- Used for unconnected sockets by explicitly specifying destination

- sendto adds additional parameters:

  - destaddress - struct sockaddr destination address

  - addresslen - length of destaddress

- sendmsg combines list of parameters into single structure:

struct msgstruct {

struct sockaddr *m_addr;   /* ptr to destination address */

struct datavec  *m_vec;    /* pointer to message vector  */

int    m_dvlength;         /* num. of items in vector    */

struct access   *m_rights; /* ptr to access rights list  */

int    m_alength;          /* num. of items in list      */

}

recv

recv(socket, buffer, length, flags)

- Used to receive incoming data through connected socket

- socket identifies the socket

- Data copied into buffer

- At most length bytes will be recved

- flags give special options

- Returns number of bytes actually recved

  - 0 implies connection closed

  - -1 implies error

recvfrom, recvmsg

recvfrom(socket, buffer, length, flags, sndraddress, addresslen)

recvmsg(socket, msgstruct, flags)

- Like sendto and sendmsg (in reverse!)

- Address of source copied into sndraddress

- Length of address in addresslen

- recvmsg uses msgstruct for parameters

Other procedures

- getpeername - address of other end of connection

- getsockname - current address bound to socket

- setsockopt - set socket options

Sockets and processes

- Like file descriptors, sockets are inherited by child processes

- Socket disappears when all processes have closed it

- Servers use socket inheritance to pass incoming connections to slave server processes

Summary

- Socket API is de facto standard

  - Originally developed for BSD UNIX

  - Copied to many other systems

- Sockets are an extension of the UNIX file I/O system

  - Use same descriptor addresses

  - Can (but typically don't) use same system calls

- Many specific system calls for sockets

Chapter 25 - Client/Server Example

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Connection-oriented communication |
| 3 | An example service |
| 4 | Example programs |
| 5 | Program architecture |
| 6 | Server |
| 7 | Client |
| 8 | Client calls to recv |
| 9 | Sockets and blocking |
| 10 | Using client with another server |
| 11 | Using another client with server |
| 12 | Summary |

Introduction

- Will examine details of client and server programs

- Examples use socket API

- Will illustrate details of socket use

- Will also illustrate program architecture

Connection-oriented communication

- Client/server developer must choose between connectionless and connection-oriented service

  - Connectionless can be used at any time; does not provide reliability

  - Connection-oriented requires explicit connection; provides reliable data delivery

- This example will use connection-oriented transport

  - Server contacts local protocol software to accept incoming connections

  - Client establishes connection to server through client's local protocol software

- Client and server exchange data once connection is established

An example service

- Server

  - Keeps track of how many times it has been contacted

  - Returns that count in an ASCII string

- Client

  - Establishes connection

  - Waits for message from server (ASCII string)

  - Prints data from message

- Application-level protocol:

  - Syntax:

    - No headers

    - Client sends null message body

    - Server message contains ASCII string

  - Semantics - Client establishes connection; server returns ASCII string

Example programs

- Example server in server.c

  - Runs as background program

  - Takes single (optional) command line argument: port number to listen on (default is 5193)

- Example client in client.c

  - Runs as user program

  - Takes two (optional) command line arguments: name of host to contact (default is localhost) and port number to contact (default is 5193)

Program architecture

- Sequence of socket calls in server and client

Server

- Initialization:

  - getprotobyname - looks up protocol number for TCP

  - socket - creates socket

  - listen - associates socket with incoming requests

- Loop:

  - accept - accepts incoming connection

  - send - send message to client

  - close - closes connection socket

Client

- Initialization:

  - gethostbyname - looks up server

  - getprotobyname - looks up protocol port number for TCP

  - socket - creates socket

  - connect - connects to server port

- Loop:

  - recv - receives message from server

- Termination:

  - close - closes socket

Client calls to recv

- The call to recv is in a loop in the client

- But, the server send only a single message - why would multiple calls be required?

- Protocol software does not guarantee to deliver data to receiver in the same blocks as generated by the server

  - May travel in different segments

  - recv need not return as much data as was requested

- Client must make repeated calls to recv until 0 returned

Sockets and blocking

- Most socket calls are blocking

- Calling process is blocked (taken off ready queue) until socket operation completes

- Server blocks:

  - accept until new connection arrives

  - send until data delivered to protocol software

- Client blocks:

  - gethostbyname until DNS resolves server name

  - recv until message delivered from server

Using client with another server

- Other services use similar application protocol:

  - DAYTIME

  - CHARGEN

  - MOTD

- Specifying port connects to alternate services

- Example:

$ client www.netbook.cs.purdue.edu 13

Thu Feb 27 15:19:06 1997

$ client leo.eg.bucknell.edu 13

Thu Feb 27 15:19:18 1997

$ client merlin.cs.purdue.edu 13

Thu Feb 27 15:19:26 1997

$ client cs.ucla.edu 13

Thu Feb 27 12:20:28 1997

$ client uran.informatik.uni-bonn.de 13

Thu Feb 27 21:22:37 1997

$ client yotaga.psu.ac.th 13

Fri Feb 28 03:24:20 1997

Using another client with server

- Other client that uses same application protocol can test server

- Example: telnet

$ telnet www.netbook.cs.purdue.edu 5193

Trying 134.82.11.70 ...

Connected to regulus.eg.bucknell.edu.

Escape character is '^]'.

This server has been contacted 5 times.

Connection closed by foreign host.

Summary

- Example client and server

  - Connection-oriented transport

  - Very simple application protocol

- Demonstrates use of socket calls

- Can be used with other clients and servers

Chapter 26 - the Domain Name System

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Structure of DNS names |
| 3 | DNS naming structure |
| 4 | Geographic structure |
| 5 | Domain names within an organization |
| 6 | Example DNS hierarchy |
| 7 | DNS names and physical location |
| 8 | Client-server computing |
| 9 | DNS and client-server computing |
| 10 | DNS server hierarchy |
| 11 | Choosing DNS server architecture |
| 12 | Name resolution |
| 13 | DNS messages |
| 14 | DNS servers |
| 15 | Using DNS servers |
| 16 | DNS caching |
| 17 | Types of DNS entries |
| 18 | Abbreviations |
| 19 | Summary |

Introduction

- IP assigns 32-bit addresses to hosts (interfaces)

  - Binary addresses easy for computers to manage

  - All applications use IP addresses through the TCP/IP protocol software

  - Difficult for humans to remember:

% telnet 134.82.11.70

- The Domain Name System (DNS) provides translation between symbolic names and IP addresses

Structure of DNS names

- Each name consists of a sequence of alphanumeric components separated by periods

- Examples:

www.eg.bucknell.edu

www.netbook.cs.purdue.edu

charcoal.eg.bucknell.edu

- Names are hierarchical, with most-significant component on the right

- Left-most component is computer name

DNS naming structure

- Top level domains (right-most components; also known as TLDs) defined by global authority

| com | Commercial organization |
| --- | --- |
| edu | Educational institution |
| gov | Government organization |
| mil | Military organization |

- Organizations apply for names in a top-level domain:

bucknell.edu

macdonalds.com

- Organizations determine own internal structure

eg.bucknell.edu

cs.purdue.edu

Geographic structure

- Top-level domains are US-centric

- Geographic TLDs used for organizations in other countries:

| TLD | Country |
| --- | --- |
| .uk | United Kingdom |
| .fr | France |
| .ch | Switzerland |
| .in | India |

- Countries define their own internal hierarchy: ac.uk and .edu.au are used for academic organizations in the United Kingdom and Australia

Domain names within an organization

- Organizations can create any internal DNS hierarchy

- Uniqueness of TLD and organization name guarantee uniqueness of any internal name (much like file names in your directories)

- All but the left-most component of a domain name is called the domain for that name:

| Name | Domain |
| --- | --- |
| www.netbook.cs.purdue.edu | netbook.cs.purdue.edu |
| regulus.eg.bucknell.edu | eg.bucknell.edu |
| coral.bucknell.edu | bucknell.edu |

- Authority for creating new subdomains is delegated to each domain

- Administrator of bucknell.edu has authority to create eg.bucknell.edu and need not contact any central naming authority

Example DNS hierarchy

DNS names and physical location

- DNS domains are logical concepts and need not correspond to physical location of organizations

- DNS domain for an organization can span multiple networks

  - bucknell.edu covers all networks at Bucknell

  - www.netbook.cs.purdue.edu is in 318 Dana

  - laptop.eg.bucknell.edu could be connected to a network in California

Client-server computing

- Clients and servers communicate in distributed computing

  - Client initiates contact to request some remote computation

  - Server waits for clients and answers requests as received

- Clients are usually invoked by users as part of an end-user application

- Servers are usually run on central, shared computers

DNS and client-server computing

- DNS names are managed by a hierarchy of DNS servers

- Hierarchy is related to DNS domain hierarchy

- Root server at top of tree knows about next level servers

- Next level servers, in turn, know about lower level servers

DNS server hierarchy

Choosing DNS server architecture

- Small organizations can use a single server

  - Easy to administer

  - Inexpensive

- Large organizations often use multiple servers

  - Reliability through redundancy

  - Improved response time through load-sharing

  - Delegation of naming authority

- Locality of reference applies - users will most often look up names of computers within same organization

Name resolution

- Resolver software typically available as library procedures

  - Implement DNS application protocol

  - Configured for local servers

  - Example - UNIX gethostbyname

- Calling program is client

  - Constructs DNS protocol message - a DNS request

  - Sends message to local DNS server

- DNS server resolves name

  - Constructs DNS protocol message - a DNS reply

  - Sends message to client program and waits for next request

DNS messages

- DNS request contains name to be resolved

- DNS reply contains IP address for the name in the request

DNS servers

- Each DNS server is the authoritative server for the names it manages

- If request contains name managed by receiving server, that server replies directly

- Otherwise, request must be forwarded to the appropriate authoritative server

Using DNS servers

- DNS request is forwarded to root server, which points at next server to use

- Eventually, authoritative server is located and IP address is returned

- DNS server hierarchy traversal is called iterative resolution

- Applications use recursive iteration and ask DNS server to handle traversal

DNS caching

- DNS resolution can be very inefficient

  - Every host referenced by name triggers a DNS request

  - Every DNS request for the address of a host in a different organization goes through the root server

- Servers and hosts use caching to reduce the number of DNS requests

  - Cache is a list of recently resolved names and IP addresses

  - Authoritative server include time-to-live with each reply

Types of DNS entries

- DNS can hold several types of records

- Each record includes

  - Domain name

  - Record type

  - Data value

- A records map from domain name to IP address

  - Domain name - regulus

  - Record type - A

  - Data value - 134.82.56.118

- Other types:

  - MX (Mail eXchanger) - maps domain name used as e-mail destination to IP address

  - CNAME - alias from one domain name to another

- Result - name that works with one application may not work with another!

Abbreviations

- May be convenient to use abbreviations for local computers; e.g. coral for coral.bucknell.edu

- Abbreviations are handled in the resolver; DNS servers only know full-qualified domain names (FQDNs)

- Local resolver is configured with list of suffixes to append

- Suffixes are tried sequentially until match found

Summary

- Domain Name System maps from computer names and IP addresses

- Important to hide 32-bit IP addresses from humans

- DNS names are hierarchical and allocated locally

- Replication and caching are important performance enhancements

- DNS provides several types of records

Chapter 27 - Electronic Mail

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Electronic mail paradigm |
| 3 | Electronic mailboxes |
| 4 | E-mail addresses |
| 5 | Networked e-mail addresses |
| 6 | Internet mail addressing |
| 7 | E-mail message format |
| 8 | E-mail headers |
| 9 | E-mail example |
| 10 | E-mail headers |
| 11 | Data in e-mail |
| 12 | MIME |
| 13 | MIME (continued) |
| 14 | Programs as mail recipients |
| 15 | Mail transfer |
| 16 | SMTP |
| 17 | SMTP protocol exchange |
| 18 | Multiple recipients on one computer |
| 19 | Mailing lists and forwarders |
| 20 | Mail gateways |
| 21 | Mail gateways and forwarding |
| 22 | Mail gateways and e-mail addresses |
| 23 | Mailbox access |
| 24 | Mail access protocols |
| 25 | POP and dialup access |
| 26 | Summary |

Introduction

- Many user applications use client-server architecture

- Electronic mail client accepts mail from user and delivers to server on destination computer

- Many variations and styles of delivery

Electronic mail paradigm

- Electronic version of paper-based office memo

  - Quick, low-overhead written communication

  - Dates back to time-sharing systems in 1960s

- Because e-mail is encoded in an electronic medium, new forms of interaction are possible

  - Fast

  - Automatic processing - sorting, reply

  - Can carry other content

Electronic mailboxes

- E-mail users have an electronic mailbox into which incoming mail is deposited

- User then accesses mail with a mail reader program

- Usually associated with computer account; one user may have a different electronic mailboxes

E-mail addresses

- Electronic mailbox is identified by an e-mail address

- Typically user's account ID, although not always

- On non-networked multi-user computer, e-mail address is just account ID (no need to identify computer)

Networked e-mail addresses

- Mail delivery among networked computers is more complicated

- Must identify computer as well as mailbox

- Syntactically, e-mail address is composed of computer name and mailbox name

- Common example - user@host

- Other:

  - host1!host2!host!user

  - host%user

Internet mail addressing

- User portion is site-specific

  - droms

  - Ralph_E._Droms

  - 578.4309

- Host portion is domain name

- Source mail client

  - Resolves destination name using DNS (MX, if available)

  - Contacts mail delivery server at destination

  - Copies mail to server

- Destination mail server

  - Interprets user name according to local mailbox addresses

  - Places mail in appropriate mailbox

E-mail message format

- Simple two-part format:

  - Header includes delivery information

  - Body carries text of message

- Header and body separated by blank line

E-mail headers

- Lines of text in format keyword: information

- keyword identifies information; information can appear in any order

- Essential information:

  - To: list of recipients

  - From: sender

  - Cc: list of copy recipients

- Useful information:

  - Reply-to: different address than From:

  - Received-by: for debugging

- Frivolous information:

  - Favorite-drink: lemonade

  - Phase-of-the-moon: gibbous

E-mail example

E-mail headers

- Mail software passes unknown headers unchanged

- Some software may interpret vendor-specific information

Data in e-mail

- Original Internet mail carried only 7-bit ASCII data

- Couldn't contain arbitrary binary values; e.g., executable program

- Techniques for encoding binary data allowed transport of binary data

- uuencode: 3 8-bit binary values as 4 ASCII characters (6 bits each)

  - Also carries file name and protection information

  - Incurs 33% overhead

  - Requires manual intervention

MIME

- Extends and automates encoding mechanisms - Multipart Internet Mail Extensions

- Allows inclusion of separate components - programs, pictures, audio clips - in a single mail message

- Sending program identifies the components so receiving program can automatically extract and inform mail recipient

  - Header includes:

MIME-Version: 1.0

Content-Type: Multipart/Mixed; Boundary=Mime_separator

  - Separator line gives information about specific encoding

  - Plain text includes:

Content-type: text/plain

MIME (continued)

- MIME is extensible - sender and receiver agree on encoding scheme

- MIME is compatible with existing mail systems

  - Everything encoded as ASCII

  - Headers and separators ignored by non-MIME mail systems

- MIME encapsulates binary data in ASCII mail envelope

Programs as mail recipients

- Can arrange for e-mailbox to be associated with a program rather than a user's mail reader

- Incoming mail automagically processed as input to program

- Example - mailing list subscription administration

- Can be used to implement client-server processing

  - Client request in incoming mail message

  - Server response in returned mail reply

Mail transfer

- E-mail communication is really a two-part process:

  - User composes mail with an e-mail interface program

  - Mail transfer program delivers mail to destination

    - Waits for mail to be placed in outgoing message queues

    - Picks up message and determines recipient(s)

    - Becomes client and contacts server on recipient's computer

    - Passes message to server for delivery

SMTP

- Simple Mail Transfer Protocol (SMTP) is standard application protocol for delivery of mail from source to destination

- Provides reliable delivery of messages

- Uses TCP and message exchange between client and server

- Other functions:

  - E-mail address lookup

  - E-mail address verification

SMTP protocol exchange

220 coral.bucknell.edu Sendmail 5.65v3.0 (1.1.8.2/29Aug94-0956AM) Sat, 5 Apr 1997 06:47:12 -0500

HELO regulus.eg.bucknell.edu

250 coral.bucknell.edu Hello regulus.eg.bucknell.edu, pleased to meet you

MAIL FROM: droms

250 droms... Sender ok

RCPT TO: droms

250 droms... Recipient ok

DATA

354 Enter mail, end with "." on a line by itself

This is a test mail message.

.

250 Ok

QUIT

221 coral.bucknell.edu closing connection

Multiple recipients on one computer

- Suppose droms@bucknell.edu, zaccone@bucknell.edu and hyde@bucknell.edu are all recipients of a mail message

- SMTP allows client to specify all three and delvier single copy of message

- Server makes three copies for delivery

Mailing lists and forwarders

- E-mail addresses can be attached to programs as well as electronic mailboxes

- Mail exploder or mail forwarder resends copies of message to e-mail addresses in mailing list

  - UNIX mail program sendmail provides simple mail aliases

  - Mailing list processor, e.g., listserv, can also interpret subscription management commands

Mail gateways

- Mailing list processing may take significant resources in large organization

- May be segregated to a dedicated server computer: mail gateway

  - Provides single mail destination point for all incoming mail

  - e.g., bucknell.edu

  - Can use MX records in DNS to cause all mail to be delivered to gateway

Mail gateways and forwarding

- Users within an organization may want to read mail on local or departmental computer

- Can arrange to have mail forwarded from mail gateway

- Message now makes multiple hops for delivery

- Hops may be recorded in header

- Forwarded mail may use proprietary (non-SMTP) mail system

Mail gateways and e-mail addresses

- Organization may want to use uniform naming for external mail

- Internally, may be delivered to many different systems with different naming conventions

- Mail gateways can translate e-mail addresses

| Ralph_Droms | droms@bucknell.edu |
| --- | --- |
| Dan_Little | dlittle@mail.bucknell.edu |
| Ruth_Miller | miller@charcoal.eg.bucknell.edu |

Mailbox access

- Where should mailbox be located?

- Users want to access mail from most commonly used computer

- Can't always use desktop computer as mail server

  - Not always running

  - Requires multitasking operating system

  - Requires local disk storage

- Can TELNET to remote comptuer with mail server

Mail access protocols

- Instead of TELNET, use prtocl that accesses mail on remote computer directly

- TCP/IP protocol suite includes Post Office Protocol (POP) for remote mailbox access

  - Computer with mailboxes runs POP server

  - User runs POP client on local computer

  - POP client can access and retrieve messages from mailbox

  - Requires authentication (password)

  - Local computer uses SMTP for outgoing mail

POP and dialup access

- POP useful for dialup connection

  - Users computer not always connected

  - Can download all mail at once and read off-line

  - Can compose mail off-line and mail in one connection

Summary

- Electronic mail based on office memo paradigm

- Allows quick, asynchronous communication across entire Internet

- Can attach e-mail addresses to programs for processing

  - Mailing lists

  - Other client-server applications

- Simple Mail Transfer Protocol (SMTP) is Internet standard for mail delivery

- Mail gateways

  - Provide uniform user addressing outside organizations

  - Translate from Internet mail (e.g., SMTP) to proprietary systems

- Post Office Protocol (POP) allows remote access to electronic mailboxes

Chapter 28 - File Transfer and Remote File Access

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Two problems |
| 3 | Generalized file transfer |
| 4 | Interactive and batch transfer |
| 5 | File transfer Protocol |
| 6 | Model and interface |
| 7 | ftp client commands |
| 8 | Two-way file transfer |
| 9 | File name translation |
| 10 | File types and transfer modes |
| 11 | FTP messages |
| 12 | FTP client-server model |
| 13 | Using separate data connections |
| 14 | TFTP |
| 15 | NFS |
| 16 | NFS function |
| 17 | NFS implementation |
| 18 | Summary |

Introduction

- Many programs written to use disk file paradigm for I/O

- Moving a file from one computer to another required removable medium and sneakernet

- Network allows direct communication

  - File transfer - equivalent of tape, floppy transfer

  - Remote file system - access to files on networked computer through same interface as local files

Two problems

- Coordinating scheduling of distributed computations

- Saving intermediate results

- File transfer paradigm - programs write intermediate results to disk file

  - Components of distributed application need not be run concurrently

  - Intermediate results can be used to restart failed computation

Generalized file transfer

- Allow transfer of arbitrary files

- Accommodate different file types

- Convert between heterogeneous systems

  - Data types

  - Word lengths

  - Rules for file names

- User login

Interactive and batch transfer

- Batch transfer

  - User creates list of files to be transferred through interface program

  - Request dropped in queue

  - Transfer program reads requests and performs transfers

  - Transfer program retries until successful

  - Good for slow or unreliable transfers

- Interactive transfer

  - User starts transfer program

  - Actions include listing contents of directories, transferring files

  - User can find and transfer files immediately

  - Quick feedback in case of, e.g., spelling errors

File transfer Protocol

- TCP/IP standard is File Transfer Protocol (FTP)

- General purpose protocol

  - Operating system and hardware independent

  - Transfers arbitrary files

  - Accommodates file ownership and access restrictions

- Predates TCP/IP; adapted to TCP/IP later

Model and interface

- Underlying protocol can run either interactive or batch mode

  - ftp client gives interactive interface

  - MIME, HTTP can use directly

- Protocol actions include:

  - List contents of directory

  - Change to a different working directory

  - Retrieve a file

  - Put a file

ftp client commands

- ftp client interface from BSD UNIX is de facto standard

  - Many commands archaic and no longer used: tenex, carriage control

  - Most often used: cd, dir, ls, get, put

  - Other useful: cr, pwd, lcd

- Two-step process:

  - Launch ftp

  - Connect to remote host

    - Connect involves using user account on remote host

    - Some FTP servers provide anonymous FTP

Two-way file transfer

- get: from FTP server to local host

- put: to FTP server from local host

- Default uses same name on both hosts; ftp client allows specification of different names

- mget, mput transfer multiple files

  - UNIX-like wildcard expansion

  - prompt disables prompt for batch transfer

File name translation

- File name syntaxes may be incompatible

- UNIX - 128 character, mixed case; DOS - 8+3 character, upper case

- Some names may not be legal in all systems

- BSD ftp allows rules for filename translation

File types and transfer modes

- Many different styles of file typing

  - UNIX - untyped; may hold anything

  - MacOS - strongly typed

- ftp does two types of transfer:

  - Text - with appropriate translations to maintain integrity

  - Binary - no translation whatsoever

FTP messages

- Each message from server includes a three-digit decimal number

  - 226 Transfer complete

  - 221 Goodbye

- Convenient for computer and human recognition

- Verbose mode shows messages; quiet mode suppresses messages

FTP client-server model

- Remote server accepts control connection from local client

  - Client sends commands to server

  - Persists through entire session

- Server creates data connection for data transfer

  - One data connection for each transferred file

  - Data transferred (either way)

Using separate data connections

- Separates commands from data

- Client can send commands during data transfer

- Closed connection indicates end of file

TFTP

- Trivial File Transfer Protocol (TFTP) - much simpler than FTP

  - Based on UDP

  - File transfer only; no directory listing

  - No authorization

- Can be used in UDP-only system

- Requires less code than FTP

- Often used for bootstrap; e.g., ROM-based diskless system

NFS

- Network File System gives random access to files across a network

- Many distributed file systems exist; NFS is TCP/IP standard

  - AppleShare

  - Microsoft/NETBIOS

- Based on client-server model

- Originally developed by Sun; implementations now available on most UNIX and many PC systems

NFS function

- Provides functions equivalent to OS access to local files

  - Open

  - Read

  - Write

  - Close

- Local file access operations mapped to network messages

  - Each message contains file system operation

  - Read/write carry one disk block in each message

- File naming integrated into local directory system

  - Remote file systems mounted onto local directory

  - Access through mount point mapped to server

NFS implementation

- NFS operations are UNIX-like, but not exactly equivalent

  - Local OS performs some operations - open, close

  - NFS provides block read/write; local OS does buffering

- Other OS file functions can also be mapped into NFS operations

- File naming, authorization/account identification, access rights all problematic

Summary

- FTP - whole file transfer using TCP between Internet hosts

  - Directory listing

  - Data translation

- TFTP - whole file transfer using UDP between Internet hosts

  - File transfer only

  - No authorization

- NFS - file-level access using UDP

Chapter 29 - WWW

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Hypertext/hypermedia |
| 3 | Hypermedia pointers |
| 4 | Browser interface |
| 5 | Document representation |
| 6 | HTML |
| 7 | Example |
| 8 | CS363 - Example Page |
| 9 | Other HTML Tags |
| 10 | Embedded graphics |
| 11 | Identifying a page |
| 12 | Links between HTML documents |
| 13 | Client-server model |
| 14 | Server architecture |
| 15 | Browser architecture |
| 16 | Caching in browsers |
| 17 | Summary |

Introduction

- Hypertext model

- Use of hypertext in World Wide Web (WWW)

- WWW client-server model

- Use of TCP/IP protocols in WWW

Hypertext/hypermedia

- Hypermedia system allows interactive access to collections of documents

- Document can hold:

  - Text (hypertext)

  - Graphics

  - Sound

  - Animations

  - Video

- Documents linked together

  - Nondistributed - all documents stored locally (like CD-ROM)

  - Distributed - documents stored on remote servers

Hypermedia pointers

- Each document contains links (pointers) to other documents

  - Link represented by "active area" on screen

    - Graphic - button

    - Text - highlighted

  - Selecting link fetches referenced document for display

- Links may become invalid

  - Link is simply a text name for a remote document

  - Remote document may be removed while name in link remains in place

Browser interface

- Interactive, "point-and-click" interface to hypermedia documents

- Each document is displayed in screen

- User can select and follow links - "point-and-click"

- Application is called a browser (infinite time sink)

Document representation

- Each WWW document is called a page

- Initial page for individual or organization is called a home page

- Page can contain many different types of information; page must specify

  - Content

  - Type of content

  - Location

  - Links

- Rather than fixed WYSIWYG representation (e.g., Word), pages are formatted with a mark up language (like TeX)

  - Allows browser to reformat to fit display

  - Allows text-only browser to discard graphics

- Standard is HyperText Markup Language (HTML)

HTML

- HTML specifies

  - Major structure of document

  - Formatting instructions

  - Hypermedia links

  - Additional information about document contents

- Two parts to document:

  - Head contains details about the document

  - Body contains information/content

- Page is represented in ASCII text with embedded HTML tags formatting instructions

  - Tags have format <TAGNAME>

  - End of formatted section is </TAGNAME>

Example

<HTML>

<HEAD>

<TITLE>

CS363 - Example Page

</TITLE>

</HEAD>

<BODY>

<HR>

Lecture notes for today go here!

<HR>

<CENTER>

<TABLE BORDER=3>

<TR>

<TD><A HREF="./chap27_6.html">Previous</A>

<TD><A HREF="./chap27_8.html">Next</A>

<TD><A HREF="./chap27_0.html">Top of chapter</A>

<TD><A HREF="../lecture_notes.html">Lecture notes</A>

<TD><A HREF="../../index.html">Top of CS363</A>

</TABLE>

</CENTER>

</BODY>

</HTML>

Other HTML Tags

- Headings - <H1>, <H2>

- Lists

  - <OL> - Ordered (numbered) list

  - <UL> - Unordered (bulleted) list

  - <LI> - List item

- Tables

  - <TABLE>, </TABLE> - Define table

  - <TR> - Begin row

  - <TD> - Begin item in row

- Parameters

  - Keyword-value pairs in HTML tags

  - <TABLE BORDER=3>

Embedded graphics

- IMG tag specifies insertion of graphic

- Parameters:

  - SRC="filename"

  - ALIGN= - alignment relative to text

- Image must be in format known to browser, e.g., Graphics Interchange Format (GIF)

Identifying a page

- Page identified by:

  - Protocol used to access page

  - Computer on which page is stored

  - TCP port to access page

  - Pathname of file on server

- Specific syntax for Uniform Resource Locator (URL): protocol://computer_name:port/document_name

  - Protocol can be http, ftp, file, mailto

  - Computer name is DNS name

  - (Optional) port is TCP port

  - document_name is path on computer to page

Links between HTML documents

- Each link is specified in HTML

- Item on page is associated with another HTML document

- Link is passive; no action taken until selected

- HTML tags are <A> and </A>

  - Linked document specified by parameter: HREF="document URL"

  - Whatever is between HTML tags is highlighted item

- <A HREF="http://www.bucknell.edu/~droms">Your obdt. svt.</A>

- Your obdt. svt.

Client-server model

- Browser is client, WWW server is server

- Browser:

  - Makes TCP connection

  - Sends request for page

  - Reads page

- Each different item - e.g., IMG - requires separate TCP connection

- HyperText Transport Protocol (HTTP) specifies commands and client-server interaction

Server architecture

- Much like robowar or ftp server

  - Waits for incoming connection

  - Accepts command from connection

  - Writes page to connection

- Performance is hard issue

Browser architecture

- Browser has more components:

  - Display driver for painting screen

  - HTML interpreter for HTML-formatted documents

  - Other interpreters (e.g., Shockwave) for other items

  - HTTP client to fetch HTML documents from WWW server

  - Other clients for other protocols (e.g., ftp)

  - Controller to accept input from user

- Must be multi-threaded

Caching in browsers

- Downloading HTML documents from servers may be slow

  - Internet congested

  - Dialup connection

  - Server busy

- Returning to previous HTML document requires reload from server

- Local cache can be used to hold copies of visited pages

- Also can implement organizational HTTP proxy that caches documents for multiple users

Summary

- WWW is based on hypermedia

- HTML is markup language for WWW documents

- HTML can specify links to other documents

- WWW based on client-server model

  - Browser - client

  - WWW server - server

Chapter 30 - CGI and Dynamic Web Documents

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Document types |
| 3 | Dynamic documents and servers |
| 4 | CGI standard |
| 5 | Output from CGI program |
| 6 | CGI example |
| 7 | Inputs to CGI programs |
| 8 | State information |
| 9 | CGI program with long-term state |
| 10 | CGI program with short-term state |
| 11 | Forms and interactions |
| 12 | Summary |

Introduction

- Documents in previous section are static

  - Defined in text file by page author

  - Remains unchanged until edited by author

- Dynamic documents are generated on demand by HTTP server

- Active execute code on the WWW browser host computer

Document types

| Document type | Description | Advantages/disadvantages |
| --- | --- | --- |
| Static | - Text file on server - Edited by author - Contents unchanged (until edited) | - Easy to generate - Fast to display - Inflexible |
| Dynamic | - Program on server - Output returned to client - Client displays as HTML page | - Can generate contents at time of access - Harder to author - Must be refreshed to update display |
| Active | - Consists of a program that is downloaded to browser - Browser executes program - Interacts with user through keyboard - Updates display - Can read files and contact other Internet services | - Browser must be able to run program - Program must be platform independent - Requires more complexity in browser - Standards not fixed (Java is de facto standard - Requires programming skill (typically more than dynamic) |

Dynamic documents and servers

- Support for dynamic documents requires some changes on HTTP server

- Typically implemented as extensions to existing server

- Server must be able to execute program that generates dynamic document

  - Can be implemented in

    - Shell/csh script

    - C program (executable)

    - Others?

  - Output returned to browser (looks like contents of static page)

- Each dynamic document needs separate program

- Server must differentiate between static and active document references

CGI standard

- Common Gateway Interface (CGI) standard defines server-application interaction

- Program sometimes called CGI program

- Guidelines for interaction

  - Language

  - Parameter passing

Output from CGI program

- Output from CGI program routed to WWW browser

- Can be in several formats, e.g., plain text or HTML

- Program must identify content to server for relay to browser

- Header - consists of lines of text followed by blank line

Content-type: text/html

Content-type: text/plain

Location: http://other.server.com/new.txt

CGI example

- Here's an example

A simple CGI program

- Try it again...

- Uses shell script language

  - echo prints its command line arguments

  - date returns date

  - `foo` returns output from command foo as command line argument

- Linked to with:

<a href="http://www.netbook.cs.purdue.edu/cgi-bin/netbook-28-1">

A simple CGI program</a>

Inputs to CGI programs

- Server passes inputs to CGI programs through parameters

- Browser may supply parameters (through server)

  - Browser extends URL with additional parameters

  - Separated by "?"

- Parameters passed as environment variables

  - Essentially keyword-value pairs

  - Based on original UNIX servers

- Very easy to access in shell script

State information

- CGI program invoked identically at each reference

- Server maintains no history of previous invocations or any other state

- Long-term - store in file on server

- Short-term - return to browser in URL

CGI program with long-term state

- Keeps record of IP addresses

- Responds with initial message at first reference; subsequent message otherwise

- Try me!

- Try it again...

- Implementation details

  - List of known addresses kept in file ipaddrs

  - grep -s returns TRUE if argument string not found in file

  - Environment variables $REMOTE_ADDR contains string with IP address of browser host

CGI program with short-term state

- Trick is to encode state in URL, return URL in link

- When user selects link, state returned as parameter suffix in new URL

- Script can parse $QUERY_STRING to retrieve state

- Try me!

- Try it again...

- Implementation details

  - N is internal variable

  - case statement allows for null string

  - ;; is end of case

Forms and interactions

- CGI includes support for forms

- Server sends document to browser identified as a form

- Form includes items for information entry

  - Each form item has a name

  - Contents of that item encoded in query string:

?ITEM1=4,ITEM2=Bucknell,ITEM3=CS363

Summary

- Three types of WWW documents

  - Static

  - Dynamic

  - Active

- Dynamic document

  - Generated by program on server at each reference

  - May display different contents over time

  - Must be refreshed to update display

- May include long-term and short-term state

Chapter 31 Java Technology For Active Web Documents

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Continuous update through server push |
| 3 | Active documents |
| 4 | Representing and executing active documents |
| 5 | Java |
| 6 | Java language |
| 7 | Java run-time environment |
| 8 | Portability |
| 9 | Java library |
| 10 | AWT graphics |
| 11 | Java and browsers |
| 12 | Compiling a Java program |
| 13 | An example applet |
| 14 | A Java example |
| 15 | Running the example |
| 16 | Interacting with the browser |
| 17 | Running the example |
| 18 | Alternatives |
| 19 | JavaScript example |
| 20 | Summary |

Introduction

- Active documents consist of code executed on computer running browser

- Java language allows development of active document programs

  - Programs called applets

  - Platform independent

  - Secure

Continuous update through server push

- Some servers will send new versions of document

- Technique called server push

- Each server push document requires dedicated server resources

- May scale with number of clients

- Also requires network bandwidth for each update

Active documents

- Delegates responsibility for updates to browser client

- Work scales with number of active documents on client

- Active document can, in fact, incur less server overhead than dynamic document

Representing and executing active documents

- Requires programming language; what should that language look like?

- How should the language be represented for execution?

- Multiple implementations possible; standards necessary for coordination

Java

- Technique for writing, compiling, downloading and executing active documents

- Includes:

  - Programming language

  - Runtime environment

  - Class library

Java language

- Resembles C++

  - Object-oriented

  - Some C++ cruft excised or restricted

- Characteristics:

  - High level, general purpose, object oriented

  - Dynamic

  - Strongly typed, static type checking

  - Concurrent

Java run-time environment

- Interpretive execution - Java language compiled into bytecodes

- Automatic garbage collection

- Multithreaded execution

- Internet access

- Graphics

Portability

- Java must be platform-independent

- Run-time environment clearly defined and has no implementation dependencies

- Bytecode representation is platform independent

Java library

- Library provides collection of common functions for Java applets

- Class definitions and methods

- Classes:

  - Graphics

  - Low-level network I/O (socket-level)

  - Web server interaction

  - Run-time system calls

  - File I/O

  - Data structures

  - Event capture - user interaction

  - Exception handling

AWT graphics

- Java graphics library called Abstract Window Toolkit (AWT)

- Includes high-level and low-level facilities

  - Windows with components - scrollbars, buttons

  - Blank rectangular area with object drawing

- Implements abstract functions; run-time environment implementation maps to window system-specific functions

Java and browsers

- Browser must include Java interpreter

- Java interpreter works through browser

  - Graphics

  - HTML

- Interpreter also works through native operating system

  - File I/O

  - Network operations

Compiling a Java program

- javac translates Java source code into bytecodes

  - Checks for correct syntax

  - Imports classes from library

  - Writes bytecode program to filename.class

- Other development environments exist

  - May include source code management

  - "Visual" systems provide "pluggable" modules

An example applet

### NerdClock

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap31/chap31_13.html

Shockwave

A Java example

- Example code...

Running the example

<applet code="clickcount.class" height=50 width=400> </applet>

Shockwave

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap31/chap31_15.html

Interacting with the browser

Running the example

http://www.eg.bucknell.edu/~cs363/lecture_notes/chap31/chap31_17.html

Alternatives

- JavaScript

  - Interpreted scripting language

  - Like csh for browser

- Other languages compiled into Java bytecodes

- Other programming technologies - Inferno

JavaScript example

<SCRIPT LANGUAGE="JavaScript">

<!--

var loc = location.href.toString()

var ep = loc.lastIndexOf("/")

var dirloc = ""

if (ep > 0) {

dirloc = loc.substring(0, ep)

}

function doform(form)

{

// Find next file

var URL

URL = dirloc + "/page20b.htm"

// Some problems getting background to look nice...

parent.frames[2].document.open()

parent.frames[2].document.clear()

parent.frames[2].document.writeln('<HTML><HEAD></HEAD><BODY BGCOLOR="#FFFFFF">')

parent.frames[2].document.writeln("<DL>")

//search stuff goes here

URL = URL + "?" + form.keyword.value

parent.frames[0].location.href = URL

}

// some hackery to force the bottom frame to be empty every

// time the page is reloaded

var URL

var loc = location.href.toString()

var ep = loc.lastIndexOf("/")

URL = dirloc + "/page20y.htm"

// force frame 0 to empty frame so re-execution is a no-op

parent.frames[0].location.href = URL

// -->

</SCRIPT>

Summary

- Active documents execute code in browser on user's computer

- Java is most widely used active document technology

- Java consists of:

  - Programming language

  - Run-time environment

  - Class library

  - Tags in browser for Java program invocation

Chapter 32 - RPC and Middleware

| ## Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Tools for networked applications |
| 3 | Programming with procedures |
| 4 | Procedure call graph |
| 5 | Remote Procedure Call |
| 6 | RPC paradigm |
| 7 | RPC call graph |
| 8 | What does RPC mechanism have to do? |
| 9 | Where is this work done? |
| 10 | External data representation |
| 11 | Middleware and Object-oriented Middleware |
| 12 | ONC RPC |
| 13 | DCE RPC |
| 14 | CORBA |
| 15 | Summary |

Introduction

- Client-server model most often used for networked applications

  - Fits well with program execution model

  - Modular

- May not be easy to program

  - Model not intuitive to coding experience

  - Lots of details to manage

Tools for networked applications

- Lots of details to manage

  - Connections between components of networked application

  - Synchronization

  - Robust data exchange

  - Data conversions

  - Error conditions

- Many details similar or identical in different networked applications

- Idea: use tools to handle routine parts of interface

Programming with procedures

- Modularizes code

  - Reusable components

  - Procedures operate on parameters

- Procedures may call other procedures

Procedure call graph

Remote Procedure Call

- Remote Procedure Call (RPC) model provides structure for development of networked applications

  - Based on procedure call model in familiar languages

  - Related to client-server model

- Hides details of communication and synchronization beneath procedure call interface

RPC paradigm

- Network communication hidden by procedure calls

  - Procedure may be executed on different computer

  - RPC mechanism hides details

- Programmer decides which procedures are executed where

RPC call graph

What does RPC mechanism have to do?

- Caller:

  - Marshal arguments

  - Transmit procedure identifier and arguments to remote procedure

  - Wait for response

- Called procedure:

  - Unpack arguments

  - Execute procedure

  - Reply with any returned value

Where is this work done?

- Client stub with name of remote procedure on client

- Server stub "spoofs" caller on server

External data representation

- Machine-dependent data representations

  - Integer byte ordering

  - Floating point

  - Character strings (ASCII - EBCDIC; null-terminated - length encoded)

- External data representation: common format for data exchange

Middleware and Object-oriented Middleware

- Middleware: tools for generating RPC-based applications

- Interface Definition Language (IDL)

  - Defines specifications for procedure interfaces

  - Used by client and server programmers

- Object-oriented middleware: remote invocation of object methods

ONC RPC

- Open Network Computing Remote Procedure Call (ONC RPC)

  - Designed by Sun Microsystems

  - Early example of middleware

- Used in many Sun applications; e.g. NFS

- Includes eXternal Data Representation (XDR) standard

DCE RPC

- Open Software Foundation (OSF) defined Distributed Computing Environment (DCE)

- Includes DCE RPC as middleware component

- Defines its own IDL

- Microsoft derived Microsoft Remote Procedure Call (MSRPC) from DCE RPC

CORBA

- Common Object Request Broker Architecture (CORBA)

  - Developed by Object Management Group (OMG)

  - Vendor-independent, interoperable spec for middleware

- Remote invocation instantiated by local object proxies

  - Proxies instantiated at runtime

  - Methods passed to "real" remote object

Summary

- Client-server programming may be difficult due to details and synchronization

- Procedure call model is "natural" paradigm for programmers

- Remote Procedure Call (RPC) uses procedure call paradigm for client-server communication

  - Client sends parameters for procedure call to server

  - Server executes procedure and replies with returned value

- Middleware is category of development tools for RPC

Chapter 33 - Network Management: SNMP

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Internet Management |
| 3 | Types of problems |
| 4 | Problem with hidden failures |
| 5 | Network management software |
| 6 | Network management model |
| 7 | SNMP |
| 8 | SNMP data representation |
| 9 | Fetch-store paradigm |
| 10 | SNMP operations |
| 11 | Identifying objects with SNMP |
| 12 | Storing ASN.1 numeric values |
| 13 | Storing ASN.1 lengths |
| 14 | Types of MIBs |
| 15 | Arrays in MIBs |
| 16 | Array example |
| 17 | Summary |

Introduction

- Network management is a hard problem

- Will discuss network management paradigm base on network communication and client-server model

- SNMP is TCP/IP standard

Internet Management

- Network manager or network administrator is responsible for monitoring and controlling network hardware and software

  - Designs and implements efficient and robust network infrastructure

  - Identifies and corrects problems as they arise

  - Must know both hardware and software

- Why is network management hard?

  - Most internets heterogeneous

  - Most internets large

Types of problems

- Catastrophic

  - Fiber broken by backhoe

  - LAN switch loses power

  - Invalid route in router

  - Easiest to diagnose

- Intermittent or partial

  - NIC sends frames too close together

  - Router has one invalid entry

  - Hardest to diagnose

Problem with hidden failures

- Some intermittent of partial failures may not be evident to user

  - Hardware may drop frames with data errors

  - Network protocols may recover from lost packet

- However, network performance decreases

Network management software

- Monitor operation and performance of network devices:

  - Hosts

  - Routers

  - Bridges, switches

- Control operations through rebooting, changing routing table entries

Network management model

- Network management does not have an internet or transport layer protocol

- Defines application layer protocol using TCP/IP transport layer protocol

- Based on client-server model; names changes

  - Manager == client; run by network manager

  - Agent == server; runs on managed device

- Manager composes requests for agent; agent composes response and returns to manager

SNMP

- TCP/IP standard is Simple Network Management Protocol (SNMP)

- Defines all communication between manager and agent

  - Message formats

  - Interpretation of messages

  - Data representation

SNMP data representation

- SNMP uses Abstract Syntax Notation.1 (ASN.1)

  - Platform-independent data representation standard

  - Strongly-typed

  - Can accommodate arbitrary data types

- Example - integer representation

  - Length octet - number of octets containing data

  - Data octets - value in big-endian binary

Fetch-store paradigm

- Manager-agent interaction based on fetch-store paradigm

  - Fetch retrieves a value from the agent

  - Store changes a value on the agent

  - Any other information is extracted from the fetched data and displayed by the manager

- Fetch used to monitor internal data values and data structures

- Store used to modify and control data values and data structures; also used to control behavior by setting "reboot" object

SNMP operations

- Get (fetch) retrieves value of object

- Set (store) stores new values into object

- Get-next retrieves next object (for scanning)

Identifying objects with SNMP

- SNMP is not tied to any particular set of data structures

- Operates on a collection of related objects identified in a Management Information Base (MIB)

- Objects in a MIB are identified by ASN.1 naming scheme

  - Hierarchical naming structure

  - Authority for new names delegated as in DNS

- Example - count of incoming IP datagrams:

iso.org.dod.internet.mgmt.mib.ip.ipInReceives

- For efficiency, each name has a numeric equivalent; e.g.:

1.3.6.1.2.1.4.3

Storing ASN.1 numeric values

- Value stored in sequence of octets

- Leftmost bit is 0 in last octet

- Example:

Storing ASN.1 lengths

- Leftmost bit 0 means length in same octet

- Leftmost bit 1 means length in k octets

Types of MIBs

- Very flexible structure

- MIBs defined for protocols, devices, network interfaces

- MIB I is original TCP/IP standard for protocol suite; MIB II extends that original version

Arrays in MIBs

- Some types of data - such as a routing table - is most naturally stored as an array

- ASN.1 supports variable length, associative arrays

  - Number of elements can increase and decrease over time

  - Each element can be a structured object

- Indexing is implicit

  - Manager must know object is an array

  - Manager must include indexing information as suffix

Array example

- Routing table is an array:

ip.ipRoutingTable

- List of routing table entries is indexed by IP address

- To identify one value:

ip.ipRoutingTable.ipRouteEntry.ipRouteNextHop.IPdestaddr

  - ipRouteEntry indicates indexing

  - ipRouteNextHop is a field in a routing table entry

  - IPdestaddr is 32-bit IP address

Summary

- TCP/IP includes SNMP as network management protocol

- SNMP is an application protocol that uses UDP for transport

- Based on fetch-store paradigm

  - Controls operation as side-effect of store operations

  - Get-next used top scan objects

- Management Information Base (MIB) defines structure of objects

- Abstract Syntax Notation.1 (ASN.1) used for data representation and object identification

Chapter 34 - Network security

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Secure networks and security policies |
| 3 | Components of security policy |
| 4 | Aspects of security |
| 5 | Responsibility and control |
| 6 | Integrity mechanisms |
| 7 | Encryption and privacy |
| 8 | Public key encryption |
| 9 | Digital signatures |
| 10 | Digital signatures and privacy |
| 11 | Packet filtering |
| 12 | Internet firewall |
| 13 | Summary |

Introduction

- Routers forward packets - from any source

- Bad guys can send in packets from outside

- How to avoid security breaches?

Secure networks and security policies

- Can't describe a network as secure in the abstract

- University may have different notion of security than military installation

- Must define a security policy

- Many possibilities to consider:

  - Data stored on servers

  - Messages traversing LANs

  - Internal or external access

  - Read/write versus read-only access

Components of security policy

- Describes items to be protected and rules for protection

- Must cover computer systems, LANs, interconnection devices, ...

- Development must include assessment of cost of protected information versus cost of protection

Aspects of security

- Data accessibility - contents accessible

- Data integrity - contents remain unchanged

- Data confidentiality - contents not revealed

Responsibility and control

- Must be able to delegate and control responsibility

- Accountability - who is responsible for tracking access to data

- Authorization - who is responsible for who access data

Integrity mechanisms

- Checksum, CRC - protects integrity

- Message digest - more robust integrity check

Encryption and privacy

- Encryption - rewrite contents so that they cannot be read without key

  - Encrypting function - produces encrypted message

  - Decrypting function - extracts original message

  - Encryption key - parameter that controls encryption/decryption; sender and receiver share secret key

- Sender produces: E = encrypt(K, M)

- Sender transmits E on network

- Receiver extracts: M = decrypt(K, E)

Digital signatures

- Goal - guarantee that message must have originated with certain entity

- Idea - encrypt with private key, decrypt with public key

- Only owner of private key could have generated original message

Digital signatures and privacy

- Can combine techniques - signed by A, private 10 B

- A forms: X = encrypt(PUBB, encrypt(PRVA, M))

- B extracts: M = decrypt(PUBA, decrypt(PRVB))

Packet filtering

- Can configure packet forwarding devices - esp. routers - to drop certain packets

- Consider example:

- Suppose 192.5.48.0 is test network and 128.10.0.0 has controlling workstations

  - Install filter to allow packets only from 192.5.48.0 to 128.10.0.0

  - Keeps potentially bad packets away from remainder of Internet

Internet firewall

- Packet filter at edge of intranet can disallow unauthorized packets

- Restricts external packets to just a few internal hosts

- Proxies forward packets through firewall after authorization

- DMZ net adds extra layer of access

- Net 10 and network address translation (NAT) boxes also add security

Summary

- Security is a problem because Internet is not owned by one entity

- Organizations can use firewalls to prevent unauthorized access

- Encryption and digital signatures can provide confidentiality and secure identification

Chapter 35 - Initialization

| Section | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Bootstrapping |
| 3 | Protocol parameters |
| 4 | Examples |
| 5 | Modes of protocol configuration |
| 6 | Why automate? |
| 7 | Methods of automated protocol configuration |
| 8 | The chicken-and-egg problem |
| 9 | RARP |
| 10 | ICMP |
| 11 | BOOTP |
| 12 | BOOTP forwarding |
| 13 | Automatic address assignment |
| 14 | DHCP |
| 15 | Address leases |
| 16 | DHCP message format |
| 17 | DHCP and DNS |
| 18 | Summary |

Introduction

- How does protocol software begin operation

- From where is the software loaded

- How are configuration parameters determined

Bootstrapping

- Generally. initialization process is called bootstrapping

- Key idea is to perform initialization in stages, each of which enables more functions

- Protocol software may be:

  - Run out of on-board PROM

  - Loaded with bootstrap program from disk

  - Loaded with operating system from disk

Protocol parameters

- Protocol software needs specific information for operation

- Software employs parameters for operation on a specific hardware and network

- Process of supplying parameters to protocol software is called configuration

Examples

- IP address - depends on network, must be unique on network

- Default router address - where to send packets aimed at remote network

- Subnet mask - to specify if subnet addressing is used and what the subnet is

- DNS server address - for DNS queries

- Server addresses

Modes of protocol configuration

- Manual

- Local disk file

- Automated through network

Why automate?

- Centralized configuration for groups of computers

- Automatic address assignment

Methods of automated protocol configuration

- Decentralized - host discovers parameters independently

- Server-based - host contacts server

The chicken-and-egg problem

- How can host use network to get network address?

- Use broadcast-based link-layer protocol

  - RARP

  - AppleTalk

RARP

- Reverse Address Resolution Protocol (RARP)

- Maps MAC address to IP address

- Host broadcasts RARP request with its MAC address

- Server replies:

  - Must be on same subnet

  - Looks up MAC address in table

  - Replies with IP address

ICMP

- ICMP address mask request - find subnet mask

- ICMP gateway discovery - find default router

- Sequence:

- Broadcast RARP request

- Extract IP address from RARP response

- Broadcast ICMP address mask request

- Extract subnet mask from ICMP address mask reply

- Broadcast ICMP gateway discovery request

- Extract default router from ICMP gateway discovery reply

BOOTP

- Bootstrap Protocol (BOOTP) provides multiple parameters

- BOOTP request broadcast on subnet

- Server returns reply with IP address, subnet mask, servers, etc.

- Format:

BOOTP forwarding

- Maintaining BOOTP server on each subnet can be expensive

- BOOTP relay agent listens for BOOTP requests

  - Forwards to BOOTP server

  - Returns responses from server back to BOOTP client

Automatic address assignment

- BOOTP requires that client be present in server database before BOOTP arrives

- Can addresses be allocated automatically?

  - AppleTalk

  - IPX

DHCP

- Dynamic Host Configuration Protocol (DHCP) can be used to allocate addresses to hosts without pre-configuration

- Plug-and-play networking

  - Host sends DHCP request message

  - Server finds unused address

  - Adds to local list of addresses

  - Returns address to host

Address leases

- Suppose host leaves subnet?

- Address no longer in use; server should reassign

- How does server know when to reassign?

- Address is assigned with a lease

  - Client cannot use address after lease expires

  - Client can ask for extension prior to expiration

DHCP message format

- Extension of BOOTP; includes new options

DHCP and DNS

- New addresses should be entered in DNS

- DNS just recently added capability to automate entry updates

- "Soon", DHCP client or server will be able to add new entries to DNS

Summary

- Protocol software requires configuration parameters

- Small, heterogeneous networks can use decentralized configuration

- IP uses server-based configuration

  - BOOTP

  - DHCP